COMMENTS HF MODEM DEVELOPMENT

And Other Topics, e.g., ALE, Channel Simulation, etc.

Primarily From TAPR hfsig

Collected by WÝTN

Rev 7,  1-19-95

Definition of Terms

[12-22-94]

Johan,

This stuff is well covered in communications theory textbooks, but here's

a shot.

Entropy is a measure of information content. It's also a measure of

uncertainty.  Receiving a bit that you always know will be a "1" or a

"0" carries no real information; its entropy is 0 bits. A bit that is

(to you) completely unpredictable and has a 50-50 chance of being

either a 1 or a 0 carries 1 bit of entropy. A bit that is skewed, i.e.,

has some uncertainty to it but can be predicted with better than chance

odds has entropy somewhere between 0 and 1 bit.

Compression algorithms increase entropy in a data stream by removing

redundancy. An ideal compression algorithm produces output with 1 bit

of entropy per compressed data bit.

Channel capacity on an additive white gaussian noise channel is

defined by Shannon's famous formula

C = B log2(1 + S/N)

where	C = channel capacity in bits/sec ("real" bits, with 1 bit entropy each)

B = channel bandwidth in Hz

S = received signal power in watts

N = received noise power in watts

The important thing to note about the Shannon equation is the

fundamental tradeoff between bandwidth and S/N. The wider the

bandwidth, the lower the S/N required to maintain a constant

capacity. And the narrow the bandwidth, the higher the S/N required.

Since N also depends on bandwidth it's usually more instructive to

consider Eb/N0 instead of S/N:

Eb/N0 = (S/R) / (N/B) = S/N * B/R

where	Eb = received energy per bit in joules

N0 = received noise spectral density in watts/Hz.

R = data rate in bits/sec

Note that Eb/N0 is independent of signal bandwidth. Eb/N0 is still

dimensionless (just like S/N) since watts/Hz also has units of

joules. It is equal to S/N only for the case where the modulation

bandwidth in Hz equals the data rate in bits/sec. Eb/N0 is now

universally used in describing the performance of a modem against

white noise.

If you substitute this into Shannon's formula and solve for Eb/N0 you

get

Eb/N0 = B/R * (2^(R/B) - 1)

This tells you the absolute minimum Eb/N0 that any modem must have

when operating at a specified data rate R and bandwidth B. Real modems

will of course require more. In the limit of B/R = infinity, i.e.,

infinite bandwidth for a finite data rate, Eb/N0 = -1.6 dB, the famous

Shannon limit.  Note, though, that this applies ONLY to infinite

bandwidth.  For finite values of R/B, the required Eb/N0 is

higher. E.g., for R/B of unity, the minimum Eb/N0 is 0 dB.  For an R/B

of about 4 bit/sec/Hz, the minimum Eb/N0 is about +6 dB. If you make

R/B less than 1, eg, by adding FEC, the minimum required Eb/N0 drops

below 0 dB.  For a rate 1/2 code, this is about -.8 dB. Real,

implementable codes at this rate can get down to about +2.5 dB; I've

done this with soft decision sequential decoding.

Note that a "bit" represented by Eb is a real user data bit, NOT a

coded channel symbol. That's another reason to use Eb/N0: it takes FEC

into account. FEC provides a real power gain in that I can use fewer

transmitter watts to send a given number of user data bits per second.

But there's no free lunch; my transmitter power savings come at the

expense of additional bandwidth. FEC simply allows me to play with the

terms in the Shannon equation and to come more closely to its limits;

it doesn't allow me to violate it.

Power efficiency has traditionally been considered important only in

situations like deep space communications where transmitter power is

an absolute premium. But it's now getting quite a bit of attention

even where RF power is relatively cheap: the shared terrestrial

channel. That's because a modulation method that requires relatively

low Eb/N0 is not only more tolerant of interference (considering

interference to be part of the noise) but it also generates less

interference to others since it needs less power to communicate.

This is a win-win approach much like reducing weight in a car: the

lighter you make the car frame, the lighter the engine you need to

move it, which means an even lighter frame to carry the weight of the

engine, which means an even lighter engine, and so on.

It turns out that at least in cellular mobile communications, the

savings from reduced interference and increased interference tolerance

more than make up for the extra bandwidth required. This is the basis

of the CDMA spread spectrum stuff we've been developing at Qualcomm.

I'm convinced that same principles apply to *any* shared, interference

limited RF environment, including HF. John Costas said as much when he

wrote his famous paper, "Poisson, Shannon and the Radio Amateur" that

appeared in the Proceedings of the IRE, December 1959. (It has been

reprinted in many places, such as the IEEE book "Multiple Access

Communications", edited by Norman Abramson, ISBN 0-87942-292-0.)

Phil [Karn]

COLLECTED INFORMATION

Hello Charles,

Nice to hear from you. You certainly are bringing up a number of issues

that have resulted in the formation of this HFSIG - good for you!

Let us entertain a few notions and bring everyone up to speed: The HF modems

that Charles are mentioning are all in the family of standards built around

MIL-STD-188. This includes a 39 parallel-tone, 16-parallel tone, and 8-ary

FSK. The latter one is also known as the ALE format. NTIA makes a CD-ROM

test source available, primarily for testing ALE equipment. Some of these

modems are capable of 2400 bps (single channel or time-divison multiplexed)

and are not shy about fully utilizing a 3kHz bandwidth channel. Several

outfits are producing such modems, and they are quite pricey - at least out

of reach for the usual radio amateur. In my opinion - (for what its worth)

this is more or less brute force bandwidth & power to meet a specific

objective. It is my belief that it is our challenge as radio amareurs to

come up with something a bit more refined, superior in performance, and

affordable. However, these documents makes a good source of information.

The chirp modulation modems that Charles mentions, implements a form of

narrow-band SS. I have a couple of papers from the IEE on the theory and

implementation details (let me know if you are interested in the

references). If you have ever dealt with chirp and chirp-z transforms, you

will have some idea what this is all about. This kind of innovation gives me

hope that something as innovative will eventually lead to better solutions.

You are quite correct about the modern trend toward parallel modems and

clever coding, and we are heading that way too - though, hopefully, by

doing our homework first to find out what basic modulation scheme to

use - this is the whole idea behind the simulator. Hopefully we will have

this basic knowledge shortly and then be able to move on to working on

coding theory and eventually protocol.

In an earlier posting I provided a guestimate of the capabilities

of a few modulation schemes (some including coding) and shown that a

typical single-DSP platform would perhaps (just a guess) be able to handle

a x4 parallel situation (using fairly conventional modulation schemes). I

hope that we would be able to do at least as good as the CLOVER II

system. I am pleased to hear that you took the plunge with the DSP-93 - keep

us posted on your developments.

Thanks for your input - keep them coming.

73's

Johan

----------------------------------------------------------------------

On Tue, 18 Oct 1994, Johan Forrer FL wrote:

>

> our present problems. One question - does it mean that if we have found our

> efficient modulation scheme, that it is just a matter of implementing it

> on top of the SS coder/decoder? (pardon my ignorance).

>

Our wireline digital video transmission system convolved the data with a

pseudo-noise data stream, converted this to analog, and put the resulting

audio signal on the wire. On the receiving end, a correlation detector was

used to recover the data.

In an RF system the audio would simply be shifted to the desired RF

spectrum using suppressed-carrier frequency-conversion techniques. It

would be interesting to consider how one might do this up-conversion

(and down conversion on the receiving end) digitally. In the HF bands

we're talking about signals that some DSPs could handle.

Well, more food for thought anyway &:)

Hugh

------------------------------------------------------------------

Greetings All,

Let me introduce myself to the "net".

I am presently employed by the USAF at Kelly AFB, San Antonio Air

Logistics Center as Systems Manager for the Central Contracting

Contract Data Management Host (spell that office automation).  I

retired from the AF Reserve in 1993 after 26 years in the reserve

program.  For the last 14 years in the program, I was Chief of

Communications Maintenance for the 32nd Aeromedical Evacuation Group

and senior communicator in the Aeromedical Evacuation System.

During Desert Shield/Storm, I was Chief of Communications for the

Theater Aeromedical Evacuation System and stationed in Saudi Arabia.

The Aeromedical Evacuation System used HF communications exclusively

for command and control communications.

The year before Operation Just Cause, the "liberation" of Panama, the

AF held an exercise which included having a NCS at Kelly AFB and an

operational location in Panama.  This, as we were to learn, a good

practice for Operation Just Cause.  During Just Cause, we learned that

the 300 baud (Bell 102 Modem) communications we were using would *not*

hack the communications needs for Aeromedical Evacuation (AE).  We had

begun using a 300 baud communications device/terminal almost 3 years

before and it had connectivity problems from the first;  however, it

was so much better than voice (un-encrypted) communications that we

put up with the downside.

Because we fell flat with AE communications during Just Cause, our

Major Command arranged a test of high speed data capability using

Harris Corp, HF Comm Division's MIL-STD-188-110c modems and a NSA

blackbox that used the MIL-STD 39 parallel tone modem in the winter of

'90 at Scott AFB.  During the test we were able to pass 40K files over

HF at 2400 and 4800 BPS with no trouble except for *BIG* QSBs, QRN and

QRM.  We would lose chunks for the file but what we received was 100%

correct.  No ARQ was used...just straight RATT (ASCII) with ProCom as

the terminal program.

The Major Command (MAJCOM) Chief Surgeon and MAJCOM AE Commander and

my unit commander directed me to include a similar demonstration/test

during AE's summer 90 exercise at Ft Hunter-Liggett, CA.  I arranged

for Harris and SAIC/SAIT to bring equipment (the MIL-STD modems and

TEMPEST LapTop computers) to demonstrate high speed HF data

transmission.  We were networked the Navy Hospital Ship on the west

Coast, Army and Air Force Medical Units and AE units on HF and

compare the capability of our old equipment (300 baud, Bell 102) with

the new modems running TCP/IP.

SAIC/SAIT forgot that their software TCP/IP (probably KA9Q NOS) would

be required to key the transmitter.  Thus, we could only duplicate

the previous demonstration but did confirm that 2400 and 4800 BPS

data transmissions on HF were possible.  90 days after the exercise

concluded, I was in Saudi.

While in Saudi, we had the opportunity to use Harris ALE and

MIL-STD-188-110c modems.  The ALE was *NOT* suitable for our network

operations (that's another story) and because our AE nets were much

like NTS nets, I doubt that ALE will ever work on the hambands.

HF high speed data transmissions did work and work very well.  The

robustness of these modems running at 2400 BPS at 100 watts provided

better thruput than our 300 baud equipment running 1000 watts.  Some

units found that the MIL-STD modems on 100 watt transceivers worked

better that 100/300 baud RATT (ASCII) at 20 KW.

As one of the evaluators of Harris' AN/URC-119 (commercially known as

the RF-350 series) HF equipment prior to production and having keep

up my relationship with Harris technical personnel, I was privy to

the technical information on their MIL-STD-188-110c modems.  I also

had the opportunity to evaluate Rockwell-Collins HF equipment that

was similar to the Harris AN/URC-119 system.  The MIL-STD, high

speed, robust modems use external DSP technology in their modems to

achieve the modem/transmission protocol.

Almost a year ago, Lou, N5SGL, discovered some of Johan's (KC7WW)

work and he and I began exchanging E-Mail about robust high speed HF

modems using commercial-off-the-shelf (COTS) sound boards.  During my

many messages with Johan, I have come to believe that hams (those of

you with a much high level of technical competency than I) can

reproduce the MIL-STD modem/transmission protocols using a DSP/ASP DMA

soundcard.

One major question must be answered.  Do we (hams) want to simply

duplicate these proven MIL-STD modems (that cost a bundle) or roll

our own along the same line.  The MIL-STD modems BW will (just) fit

into a SSB bandpass...but do we want something that wide?  The will

run 4800 BPS w/o the Reed-Solomon (RS) coding and 2400 with RS coding.

How many BPS can we get using a 8 tone (narrow spaced tones), modem

running 100 baud and can we keep the BW down to something that hams

in general and the FCC will "buy"?  Also, the cost of the "system"

must be affordable.  Should we use Golay or RS encoding for the FEC?

Many questions to be answered.

I believe we can obtain 1200 BPS with FEC and ARQ/CRC and 2400 BSP

w/o FEC but with ARQ/CRC in a *ROBUST* modem protocol.  I truly would

like to see a TCP/IP HF WAN (SMTP only) to augment and eventually

replace the HF NTS nets.

73,

Walt/K5YFW

Snail-Mail      Walt DuBose

10909 Meadowhome

San Antonio, Texas 78230

MABELL          HP 210-696-3196

BP 210-925-6081

VHF-FM*         146.46 (w/CTCSS 146.2) / 147.06 (w/CTCSS 203.5)

UHF-FM*         446.0  (w/CTCSS 146.2)

HF SSB Mobile   7.213 LSB

HF CW (40/20)   U_CALL_IT

E-Mail k5yfw@sat.ampr.org (goes to k5yfw@k5yfw.ampr.org & k5yfw@sacdm10)

(ham stuff...un-official...but important)

E-Mail k5yfw@sacdm10.kelly.af.mil

(semi-official...all mailgroups including hfsig)

E-Mail wdubose@sacdm01.kelly.af.mil

(official U.S. Government stuff and emergency communications)

* Just in case you come San Antonio.

----------------------------------------------------------------------

k5yfw@k5yfw.ampr.org (Walter D. DuBose - K5YFW) writes:

> Greetings Charles and All,

If on any of those channels there was a

>         voice or data QSO going on, it would place its signaling tone on

>         the frequency it did not recognize it and "stepped" on the QSO.

>         If critical AE data was being passed, it had to be

>         re-transmitted.  It really had a way of ruining a QSO as the

>         signaling took many seconds of transmission time on each

>         frequency to establish a connection.

Hi Walter,

O.K I understand about the problem, The fredricks ALE controller has

a voice detect function, however as they are only using a TMS320C10

I guess it is not very effective. As you say this is a problem. I have

seen an intelligent squelch that uses a DSP in conjunction with a neural

net to recognise speech on a channel, it takes a few hours to train though.

Possibly some form of pilot tone SSB would be more suitable. Or the ALE

could send a "Is the channel free call" I guess there are a lot of diffent

things that could be done.

As far as Chelmsford is concerned no I am not a member although I have

been to the club a couple of times and I am friendly with one of the

committe members. I will pass on your greetings.

By the way I work at Mr Guglielmo's Wireless Works in Chelmsford.

As far as packet is concerned I wonder if it would be possible to

use a DSP based channel conditioner using the HDLC flags as a training

sequence to reduce the multi-path problems. Also if a data frame has to be

sent many times it may be possible to store multiple copies of the frame and

do a majority vote on the data. It should be possible to do this without

a change in the protocol. There is probably some flaw in my idea.

I do think an improved packet protocol for HF is needed, I know a lot of people

are thinking about the problem. I would favour a waveform similar to CLOVER

i.e using a small number of tones to reduce baud rate and a priority ack protocol.

As far as the English is concerned, my computer speaks Colonial English, anyway my father came from the Canadian Colony so I can just about understand you lot!!

Regards Charles.

--------------------------------------------------------------------

Barry Buelow - WA0RJT <barry@ia.net> writes:

> I note that some of the comments on ALE refer to async.

>

> This thought hasn't been a part of the discussions yet but I thought

> it would be interesting before we all run off in one direction.

>

Hello Barry,

Certainly a synchronous system should reduce calling time on air. A small

on air pool of calling frequencies could be used then after contact has been

made a QSY to another frequency could be initiated to prevent blocking of the

calling channels.  Non synchronous "ALE" stations could call asynchronously.

As we are not a military system we can rely on external time sources, in the

UK we have a transmission on 60 KHz called MSF this is an atomic time standard

you can buy a kit of parts to build a receiver for about $20. I guess WWV could

be used in the U.S. While we wait for GPS to drop in price.

The next problem is to channelise the HAM bands to allow easier handling in the database.

Another big problem is the Radios themselves, when your rig reads say 14.070Mhz what frequency is it actually on.  On SSB the dial frequency should be the frequency of the suppressed carrier. The other modes well I dont know.  Also it would be nice to disable the input filtering or have solid state switching

of the filters as when my TS850 scans I hear the relays clicking (and wearing out).

Frequency stability is becoming a big issue especially on modes like CLOVER

where you have to be within 10 or so Hz of the correct frequency and stay there.

It would be nice if all modern radios had an external frequency reference input.

Timeslots for sounding of stations would have to be arranged so as not to cause

collisions.

The modulation scheme during calling would need to be optimised for :

1) Fast recognition.

2) Robustness

3) Easy to measure S/N, BER, MP etc.

4) Measurement of path delay (this could be used to generate simple ionograms)

A spin off of this of course would be a wonderful beacon system.

All of this is perfectly possible with modern technology but I sometimes wonder if the guys using 5 watts and a hand key don't have the right idea.

ALE systems are being developed to reduce the skills required of the operator

and turn him into a simple communicator, operating is much of the fun of HAM radio.

Bring back spark!

Regards Charles

------------------------------------------------------------------

In Charles  message of 23 Oct 1994 at 0256 CDT, he writes:

> Barry Buelow - WA0RJT <barry@ia.net> writes:

> > I note that some of the comments on ALE refer to async.

> >

> > This thought hasn't been a part of the discussions yet but I thought

> > it would be interesting before we all run off in one direction.

> >

Charles and All,

Let me address some of these comments and then I'll go back to other

messages and address some others (perhaps).

I don't think the Harris, or Collins-Rockwell ALE units are concerned

withe being Async or Sync...perhaps I'm missing a thought or two

here.  The Fredericks unit is made under license from Harris thus I

would assume it is basically the same unit (cheaper parts? - cheaper

labor?).

>

> Hello Barry,

>

>  Certainly a synchronous system should reduce calling time on air. A small

> on air pool of calling frequencies could be used then after contact has been

> made a QSY to another frequency could be initiated to prevent blocking of the

> calling channels.

>  Non synchronous "ALE" stations could call asynchronously.

I believe that this would be imperative...but how many? 5, 10?  How

wide a channel....5, 1, 1.5, 3 KHz?  Who would decide where they

would be located...hams (by agreement), the FCC in the states or

other regulatory agency elsewhere or perhaps the IARU?  This would

require an accurate receiver and ALE signaling is *not* simply a

short burst.

For those not familiar with the signaling technique let me briefy

describe the process in un-technical terms.

1. The signal is a low baud rate robust signal...this could be made

faster.

2.  Your receiver is "scanning" all the ALE channels (yes...those

frequencies *would* channelize the ham bands) or only those for the

mode you wished to use.  You should be scanning at a specific rate and

if you are scanning more than one band, you need a multi-band

antenna...try this on for 40 & 20 meters.

3.  The signaling transmitter must transmit a query signal on

one channel for a time equal to one complete scan cycle of all the

possible channels.  Then it moves to the next channel and repeats the

process.

Using only four frequencies (channels) in Saudi during the Desert

Twins,  it took more than two minutes (total cycle query time) to try

and establish contact with a station if they never answered.

Can you imagine what it would be like with 10 hams on 20 meters trying

to make an ALE connect on just four voice calling frequencies.  *And*

this does not consider what would happen if someone decided to have a

QSO on the ALE channel or simply was coordinating a move to a working

frequency.  You *would* have to have DSP or sylabic (correct my

spelling if incorrect) squelch to keep from steping on a QSO.  All of

this assumes that you don't have a hidden transmitter...then the

matter really gets stickey.

Calling frequencies in the U.S. have never been used as calling

frequencies but rather "gathering" channels.  How has this worked in

other countries/parts of the world?  Has it worked any better?

Perhaps we in the U.S. are just not as regimented.

>

>  As we are not a military system we can rely on external time sources, in the

> UK we have a transmission on 60 KHz called MSF this is an atomic time standard

> you can buy a kit of parts to build a receiver for about $20. I guess WWV could

> be used in the U.S. While we wait for GPS to drop in price.

Why not include a seperate receiver in the ALE equipment if this is

needed.

>

>  The next problem is to channelise the HAM bands to allow easier handling in >  the database.

Good luck on the HF bands.  See above.

>

>  Another big problem is the Radios themselves, when your rig reads say 14.070Mhz

> what frequency is it actually on.  On SSB the dial frequency should be the

> frequency of the suppressed carrier. The other modes well I dont know.

>  Also it would be nice to disable the input filtering or have solid state switching

> of the filters as when my TS850 scans I hear the relays clicking (and wearing out).

Ah yes...you *WILL* (yes I screamed) need 10 Hz resolution and accuracy and

no drift and let me tell you as chief of communications maintenance

for the militray using the latest state-of-the-are equipment, maintaing

an accurate 10 Hz resolution drift free transceiver is *not* easy and

the equipment is *expensive*.  Tho, software might make this easier,

you would need software that was "rig" specific and assuming all rigs

could be tuned by software/hardware.

> Frequency stability is becoming a big issue especially on modes like

> CLOVER where you have to be within 10 or so Hz of the correct

> frequency and stay there.

See Above.

> It would be nice if all modern radios had an external frequency

> reference input.

This isn't really needed if the units frequency standard is accurate.

>  Timeslots for sounding of stations would have to be arranged so as not to cause

> collisions.

Rgr Rgr...see above

>  The modulation scheme during calling would need to be optimised for :

>  1) Fast recognition.

>  2) Robustness

>  3) Easy to measure S/N, BER, MP etc.

>  4) Measurement of path delay (this could be used to generate simple ionograms)

>

>  A spin off of this of course would be a wonderful beacon system.

I think this is a better solution...a world-wide systems of beacons

to automatically determine band is best with hardware and

software in the users transceiver...making it passive.

All this assumes reciprocity.

> All of this is perfectly possible with modern technology but I

> sometimes wonder if the guys using 5 watts and a hand key don't have

> the right idea.  ALE systems are being developed to reduce the

> skills required of the operator and turn him into a simple

> communicator, operating is much of the fun of HAM radio.

> Bring back spark!

Well, I wouldn't go that far...but basic CW is the basic digital mode.

73,  Walt/K5YFW@work

-------------------------------------------------------------------

Hi Barry

You have a good point here - if we were to lay the groundwork for a new

system, a synchronous approach would probably have definate

advantages. (note that synchronous refers to synchronous detection methods -

there also is synchronous symbol detection, that is something different).

In this regard: from what I gather, a GPS receiver would be an ideal

component in a SS system. Are there any provisions for us doing direct

sequence SS on HF? What would it take - STA? If we can resolve the details

on resolving full monitoring capabilities of amateur radio SS on HF (which

I believe is possible), this may be the answer to our problems.

Am I just day dreaming, or would it be possible to build a

RF spreading/despeading subsystem, say taking a 3 Khz voice channel and

utilizing a slot, say 14.000 - 14.200 Mhz?

73's Johan

------------------------------------------------------------------

wdubose@sacdm01.kelly.af.mil (WALTER (WALT) D. DUBOSE - PKT) writes:

>

>

> In Charles  message of 23 Oct 1994 at 0256 CDT, he writes:

> > Barry Buelow - WA0RJT <barry@ia.net> writes:

> > > I note that some of the comments on ALE refer to async.

> > >

> > > This thought hasn't been a part of the discussions yet but I thought

> > > it would be interesting before we all run off in one direction.

> > >

>

>         Charles and All,

>

> Let me address some of these comments and then I'll go back to other

> messages and address some others (perhaps).

>

> I don't think the Harris, or Collins-Rockwell ALE units are concerned

> withe being Async or Sync...perhaps I'm missing a thought or two

> here.  The Fredericks unit is made under license from Harris thus I

> would assume it is basically the same unit (cheaper parts? - cheaper

> labor?).

>

Hi Walt,

The sync part I was refering to was at call setup. All stations in

the net that are able to scan are listening on the same channel at any

time. This means there is no scanning section to the call. An originating

station can estimate the best channel tune and wait, at the right moment

when everyone is listening it makes the call. This also means that everyone

is listening at the most likely time to hear someone else.

Net synchronisation is vital, in an amateur environment an external sync

source can be used i.e GPS. In a military environment the network has to

self sync. Normaly this consists of starting in async operation, when one

hears a more senior node in the net that has sync you sync to him etc.

However when the net starts to get busy performance drops off dramatically.

Sync systems normally have abot 100 channels in the scan group, also every few

minutes a new set of 100 channels are allocated. I have a paper written by

a Prof Carlson in Sweden on this topic.

Regards Charles.

-------------------------------------------------------------------

Charles,

In your message of 24 Oct 1994 at 1404 CDT, you write:

> wdubose@sacdm01.kelly.af.mil (WALTER (WALT) D. DUBOSE - PKT) writes:

> >         Charles and All,

> >

> > Let me address some of these comments and then I'll go back to other

> > messages and address some others (perhaps).

> >

> > I don't think the Harris, or Collins-Rockwell ALE units are concerned

> > withe being Async or Sync...perhaps I'm missing a thought or two

> > here.  The Fredericks unit is made under license from Harris thus I

> > would assume it is basically the same unit (cheaper parts? - cheaper

> > labor?).

> >

>

> Hi Walt,

>  The sync part I was refering to was at call setup. All stations in

> the net that are able to scan are listening on the same channel at any

> time. This means there is no scanning section to the call. An originating

> station can estimate the best channel tune and wait, at the right moment

> when everyone is listening it makes the call. This also means that everyone

> is listening at the most likely time to hear someone else.

>  Net synchronisation is vital, in an amateur environment an external sync

> source can be used i.e GPS. In a military environment the network has to

> self sync. Normaly this consists of starting in async operation, when one

> hears a more senior node in the net that has sync you sync to him etc.

>  However when the net starts to get busy performance drops off dramatically.

> Sync systems normally have abot 100 channels in the scan group, also every few

> minutes a new set of 100 channels are allocated. I have a paper written by

> a Prof Carlson in Sweden on this topic.

I'd really hate to tie my HF operation to the presence of a satellite...

If I'm going to do that, I might as well get a nitch on a geosync

bird and send 19.2 data and to heck with HF.  Besides will I now have

to have two receivers, coax and antennas just to work HF.  And too,

my truck is already filled with radio equipment and antennas..I will

now need another "rig" and antenna on/in the truck and will have to

park in the hot sun to work HF...not under a cool shade tree cause I

gotta receive the satellite.

The reason many U.S. military units use HF is they don't want to

depend on a satellite...their *too easy* to knock out.

Realizing that GPS boxes are and will come down in price, and I'll

need another box for the ALE, I'll soon have a bunch of money in

equipment.

If we don't watch out, an HF rig will cost 2000 PS.

I might add...where's the KISS?

Walt/K5YFW

----------------------------------------------------------------------

>

> Hi Barry

>

> You have a good point here - if we were to lay the groundwork for a new

> system, a synchronous approach would probably have definate

> advantages. (note that synchronous refers to synchronous detection methods -

> there also is synchronous symbol detection, that is something different).

exactly what I was thinking.  tx and rx in at least freq sync and possibly

in phase sync.

>

> In this regard: from what I gather, a GPS receiver would be an ideal

> component in a SS system. Are there any provisions for us doing direct

> sequence SS on HF? What would it take - STA? If we can resolve the details

> on resolving full monitoring capabilities of amateur radio SS on HF (which

> I believe is possible), this may be the answer to our problems.

>

It seems to me that any serious improvement is hf throughput is not

going to come from an inexpensive chrome box that plugs in to the mic

and speaker jacks.

There is a cascading effect.   Good throughput requires good freq/time

stability.  99.99% of off the shelf rigs do NOT support external freq

references.

Without addressing SS for now, this leads to a total system design:

timebase, tx, rx, modem.  Consider that a good system out to be

something in the 50 to 100W range, and doesn't need all the chrome

and knobs of typical modern radios, single band or a couple of bands,

what else?

This is a rather substantial design task, but it brings all the important

technical issues into play.  Major flexibility of operation issues are

not a design requirement.

If this system cost $1000, it would be a huge bargain.

> Am I just day dreaming, or would it be possible to build a

> RF spreading/despeading subsystem, say taking a 3 Khz voice channel and

> utilizing a slot, say 14.000 - 14.200 Mhz?

>

> 73's Johan

>

Just dreaming along...

73 Barry

--------------------------------------------------------------------

On Mon, 24 Oct 1994, Johan Forrer FL wrote:

>

> As far as compatibility is concerned, their digital interface are all the

> same, however, there are some differences in audio circuitry. I have some

> experience with the Cardinal and the Orchid Soundwave 32. They both work

> just fine for DSP dev. work, though are second rate sound cards if that is

> what you are looking for. I bought the Orchid product because it uses the

Is the audio interface on these cards sufficient for digital communication?

>

> The PSA architecture was designed with the upcoming Windows 95 (Chicago)

> DSP API in mind - In the mean time, Analog Devices are giving away free SDK

> for using this card doing DSP development work under Windows. I have

> written a few programs using this platform - however, it requires yet

> another layer of programming and further knowledge of yet another

> programming philosophy to get even something simple working. It

> does work reasonably well. I do believe, however, that for our purposes, we

> would need to stick to writing in assembly and DOS (for a while at least).

>

How about Linux? With it's multitasking capabilities, Linux is an excellent

platform for packet. And, of course, for software development you can't

beat Unix. All we need are Linux driver for the Personal Sound

Architecture and Unix-hosted development tools.

> Other software that I have available for these PSA cards:

>

> * PSATOR     - AMTOR for the PSA cards (RTTY/ASCII will be added in future)

> * PSA-Pactor - Pactor for PSA cards. This is a full rx/tx implementation

>              using the adaptive modems mentioned above. It also implements

>              full 16-bit memory ARQ in conjunction with brute force search

>              algorithms. A 386/25 or better computer is required.

> * LMS noise reduction - A port of the W9GR code.

How about K9NG/G3RUH modulation? (I'm also putting together a node for our

local VHF TCP/IP network.)

73

Hugh

------------------------------------------------------------------

Greetings HFers,

I thought I'd try and re-name this thread from DEMODULATION to

Modulation.  Realizing however, that both are needed and in complex

modems and both creating the modulator and detector on a sound card is

going to be a challange.

For those who have gotten in "late", Johan and I have been discussing

robust, high speed modems for almost a year and Greg and I have been

carrying on a dialog for several months now.

I have used commercial/military equipment on HF at 2400 and 4800 BPS

that were *very* robust modems.  We even ran on-the-air tests against

SITOR (the commercial version of AMTOR) over the same channel.  The high

speed modems beat SITOR in transferring data at a factor greater than

the difference in bit rate.  That is to say that at 2400 BPS for the

robust HF modem we had 100% thruput at 2400 BPS.  It was capable of 2400

BPS data transfer rate and it transfered 2400 bps.  On SITOR, we had

less than 100% of the possible thruput rate.  In Jan of 1990, the modem

cost $15,000.  Today you can get it for $3,200.

I have seen the block diagram and chips it has and it "looks" like a

sound card and has the same type DSP & ASP chips as soundcards.  While

probably would not want to just emulate that modem, we should improve on

the design, we should make it a goal as we know it is achievable.

Many of my friends ask me why I hate CLOVER.  I tell them that I don't

but feel for that amount of money, hams should get more BPS for their

buck.  Hear's the reason.  CLOVER uses 4 parallel tones to get 800-900

BPS.  The commercial/military modems use 39 tones to get 2400 BPS with

the 40th tone as an unmodulated/standard modulated pilot/doppler tone.

I believe that some where between 4 and 40 tones hams (those of you who

are much smarter than I) can come up with a modem for HF that will be

very robust and produce 1200-2400 (and maybe more) BPS in a BW that is

less than the commercial/military modem.

If the guys who will do the actual programming "max-out" the soundcard

and produce a robust, 2400 BPS (100 baud or less) HF modem and can apply

the the same principle to a V/UHF modem and get 9600/19.2K BPS, ham

radio will have greatly benefited.  If they can produce only 1200 BPS on

HF and 4800 or 9600 on V/UHF, thats Ok too...just max-out the sound

cards capability.  By then, perhaps better sound cards and/or other

devices will be on the market that will let us go higher.

I like the picture of the frog choking the water fowl while the frog's

head in in the bird's mouth.  The caption is never give up.  Don't give

up, try hard, keep it low cost and KISS.

73 & have a nice weekend ya'll.

Walt@home (dba k5yfw)

------------------------------------------------------------------------

Hi all,

It is not quite so obvious which "magic" technology we should pursue

that will give maximum gains. Well, perhaps there is no magic answer,

or at least; those that know better won't say either or won't be

bothered. Lets face it, most everything that we are about to

embark on, have been done before in one form or another

commercially - though I am not aware of anything affordable for

amateurs to experiment with (yes, Clover could have been the

answer, except that it was developed as a "closed" architecture).

How one chooses one modulation/demodulation scheme over another,

is an extensive and interesting topic by itself. Without going

into much details, let me take an easy way out and go by what

others have achieved, that is, m-FSK to approx 300 bps and (nxm)-

PSK for higher rates. These have been found to be quite usable on

HF.

The m-FSK option, i.e. a system similar in modulation and coding

than what is presently implemented for ALE, would be quite do-

able on our present DSP platforms. Anyone interested in that?

Nxm-PSK is a multitone approach. In this scheme, there are

several (n- of them) independent m-PSK carriers. As far as

development is concerned, one can concentrate your development

efforts on getting the most out of one carrier, then add

additional carriers as far as your DSP horsepower will take you,

or perhaps until you run out of bandwidth (suspect the DSP is the limiting

factor). The parallel deal is to be used to get maximal bit rate as conditions

permit, or serve for additional redundancy when conditions are not so good.

This basically is the premise used for the MIL-STD-188 parallel modems. I would (rather bravely) like to suggest that this alternative be pursued - unless of course you could suggest a better alternative. Please let me know your opinion on this one.

Assuming the nxm-PSK approach in its simplest form, how does it sound if we

develop a single m-PSK channel and see how far that takes us. ?

I would suggest as background, obtaining the following three

papers (these are heavy duty, but quite good):

1) Foschini, G.J., R.D. Gitlin, and S.B. Weinstein. "On the

Selection of a Two-Dimensional Signal Constellation in the

Presence of Phase Jitter and Gaussian noise." Bell System Tech.

Jour. Vol. 52(6) July-August 1973 pp:927-965.

2) Simon, M.K., and W.C. Lindsey. "Optimum Performance of

Suppressed Carrier Receivers with Costas Loop Tracking". IEEE

Trans. on Communications Vol. COM-25(2) February 1977 pp:215-227.

A more complete and realistic treatment of the performance of

different modulation schemes and of phase lock loops in the

presence of noise and phase jitter are also given in the above

authors' book: "Telecommunications Systems Engineering." Dover

pubs. ISBN 0-486-66838-X.

3) Simon, M.K. and J.G. Smith. "Carrier Synchronization and

Detection of QASK Signal Sets." IEEE Trans. on Communications

Vol. COM-22(2) February 1974.

It is evident, at least to me, that carrier phase synchronization

and amplitude equalization becomes quite important when dealing with

these complex (meaning quadrature) waveforms. The idea of using a separate

"pilot" channel that carries such phasing and clock data, seems rather attractive. Not only will it simplify implementation details, but probably allow us to be able to track multipath effects. The additional robustness in system

performance that it would add, would offset its small amount of additional bandwidth. Let me have your views on this one too. Also think a bit about what information need to be encoded in the "pilot" carrier.

73's

Johan

PS: Thanks to Walt for forwarding a wealth of details over an extended period on the MIL-STD-188 modems. We certainly have beaten its possibilities to death.  The use of the pilot tone, or doppler channel as they call it, is used in both the 39-tone as well as the 16-tone modems.

I must also thank Bill Coleman, AA4LR, for discussions on TELEBIT's PEP, and the use of the pilot tone for dealing with multipath effects.

-----------------------------------------------------------------------

Hi Folks,

Quiet on the homefront?

A few general comments: I have suggested some reading materials -

If you have do not have access to these, perhaps I can help make

photocopies - please e-mail me directly. These papers are quite

old, though very appropriate. Unfortunately they also are quite

mathematically inclined - however, don't be intimidated by a bit

of algebra. If you are not interested in all the gory details,

just make sure you understand the premise and note the

conclusions. Some things take a while to sink in and as one

develop your grasp of the subject, it becomes clearer later.

The insight that these papers attempt to provide is: given some

model of the "channel", how does different modulation schemes

stack up? Typically various forms of OOK (on-off keying), FSK,

and PSK, are considered against white (additive Gaussian white)

noise - where it is shown that: for the same amount of emitted

energy,  phase modulation schemes have slightly lower error

rates. Now keep in mind that some rather important assumptions

are made, i.e. that we have the RX and TX in perfect sync as far

as carrier phase and symbol sync is concerned. Well, what happens

if that is not the case and what if the channel model includes phase jitter?

We see that performance drops rather dramatically. This is where the issue

of robustness comes in and where we see that noncoherent FSK actually does

very well - it requires little or no assumptions! However, just keep those

theoretical rankings in mind as the limits of performance under

ideal conditions.

Well, then why don't we just go ahead and use noncoherent FSK?

Lets say we are aiming for 1200 bps (uncoded).

Theoretically, this would require a minimum of 1200 Hz shift, and need at

least an additional 1200 Hz to accommodate the modulation

envelope - total about 2400 Hz. Two major problems: that's a lot

of bandwidth for HF, second, such short bit times would be a

disaster on HF with multipath and the other kinds of channel

anomalies. OK - so what else can we do? We have to make a

tradeoff: lower robustness for more bandwidth efficiency. That is

why some of those papers looked at those complex modulated schemes, i.e.

QAM etc.. and their performance under non-ideal conditions. The Foschini

paper surprisingly shows that QAM does better than expected -

which is great news.

The other papers is about carrier and data recovery using phase

lock loops - more about this later.

One last note: the 2-3 dB loss in performance associated with a

bandwidth-efficient modulation scheme, may be offset with coding gain

achieved by coding theory, i.e. trellis modulation/viterbi demodulation

etc. This sophistication, unfortunately, adds greater complexity to the

implementation. The DSP/host algorithms are necessarily going to be closely

coupled and this alone hints that an involved system is in your

future.

A bit of progress with the PSK modem - the modulator is working

nicely and soon I'll be testing with two sound cards back-to-

back. This setup, however, is very experimental and is, for me at

least, a way of learning how all these intricate algorithms work

together.

I have collected and studied some further materials on the channel

simulator, enough to see that it is quite doable. More about that later.

I would like to hear any further thoughts or suggestions from your side.

Please let us hear from you.

73's

Johan

------------------------------------------------------------------

Those of you who have been discussing HF channel simulation may

be interested in the following paper.  I ran across it while doing

research on an unrelated topic.

L. Ehrman, L.B. Bates, J.F. Eschile, J.M. Kates, "Realtime

Software Simulation of the HF Radio Channel" IEEE Trans. on

Communications, August 1982, p.1809

It contains an overview of the theoretical HF channel model and

details on how it was implemented on a signal processor.  They

used special purpose hardware but today's DSP chips should be up

to the task.

For those of you interested in how various modulation methods

hold up under interference, check out this paper.

O.A.H. Shabsigh, "On the Effects of CW Interference on MSK

Signal Reception" IEEE Trans. on Communications, August

1982, p.1925

Wouldn't want our fancy new demodulator to crumble when some lid

starts doing 5 WPM on top of our signal would we.

I have enjoyed lurking on this SIG for a few weeks.  Lots of

good ideas floating around (now, if we could only catch them).

By the way, I'm currently pursuing an MSEE in communications

and radar systems.  I got into amateur radio to put some of this

theory into practice.  Later!

Eric

.:::.  .:::.  .:::.

:'     :'     :'              Eric E. Silbaugh

:::.   :::.    ':.           esilbaug@afit.af.mil

::     ::         ':.

::     ::           ::   All standard, non-standard, and

'::::::::::::::::::'       MIL-STD disclaimers apply

------------------------------------------------------------------------

For those who may be interested, you can get the complete

listing of CCIR recommendations, and the text (with figures) of some of

them electronically.  The following gopher is available.  I accessed it

recently, and there's a wealth of information on line.  This are the

instructions I downloaded from the README file....

>

> You can access the ITU/TIES information via gopher:

>

>       - use your own gopher client and access to info.itu.ch on port 70

>             or

>       - telnet to gopher.itu.ch on port 2000

>

> Please send any message to helpdesk@itu.ch

>

>                                                 The TIES group

>

A search indicates that the most current version of CCIR 549 is -1, and it is

about sidetone on handset for Shipboard radiotelehpone (there was a

reference to this by one post recently).  Not sure if I copied the number

correctly or not!  The other one, was 520-2, and -2 is the current version.

It is on HF channel simulation.  Unfortunately it is not available

electronically.

- Tom, N5EG

------------------------------------------------------------------------

Date: 10 Nov 1994 05:42:37 GMT

From: karn@servo.qualcomm.com (Phil Karn)

Subject: Pulsed BPSK hoax?(???)

In article <CyzAwp.GA4@srgenprp.sr.hp.com>, alanb@hpnmarb.sr.hp.com (Alan Bloom) writes:

|> System Bartender (matt@plab.dmll.cornell.edu) wrote:

|> : Just ran across an interesting pair of articles... "A New Pulsed

|> : Bi-Phase Digital Communications System" by K6HH (73 Amateur Radio,

|> : June-July 1988).  Does anybody know anything about this proposal?

|>

Sounds familiar; I think I read it when it came out.

|> : A quick synopsis: BPSK is performed on an RF signal, but with TINY

|> : (0.02 radian = 1.2 degrees), nearly instantaneous phase shifts...

|>

|> : Ok, this seems physically feasible... but his performance claims for

|> : the circuit, namely 19.2kbit/s in < 3kHz bandwidth, seem fishy at

|> : best.

Yeah. Even if it worked, the power efficiency would be *extremely*

poor.  Almost all of his transmitted energy goes into the carrier,

which carries no information. BPSK is normally an antipodal modulation

technique, which means that the two possible symbol vectors (0 and 180

degrees) are maximally distant, and for random data there is no

discrete carrier component. This give you the best possible protection

against additive noise: a 3dB improvement against an orthogonal

modulation method such as coherently demodulated FSK with sufficient

mark/space separation. Because the two symbol vectors in this

"narrowband BPSK" scheme are so close together, there is very little

protection against noise.

This guy could have benefited from reading a basic communications

theory textbook. But as absurd as it is, his scheme is still useful in

illustrating a counter-intuitive yet fundamental principle that still

isn't widely appreciated. That is, minimizing bandwidth is exactly the

*wrong* thing to do to when you're trying to maximize spectrum

efficiency. That's because the more "bandwidth efficient" you make

your signal, the more susceptible to noise and interference it MUST

become.  Much more power is then needed to compensate for the noise

and interference, which means even worse interference to everybody

else on the band.

Minimizing *power* is what really counts, because power, not

bandwidth, is what causes interference. We should get rid of all

modulation bandwidth limits in amateur radio (as long as you stay

within the band) and replace them with a requirement for automatic

transmitter power control.

John P. Costas, K2EN and the inventor of the Costas Loop, wrote a

classic paper in the late 1950s called "Poisson, Shannon and the Radio

Amateur" in which he laid out the theoretical foundation for this

approach. He was only 3 decades before his time.  Only now has the

technology finally become capable of implementing his vision.

His thesis was that the whole concept of frequency coordination is

doomed to failure in the real world, and he held up amateur radio as

an example. The only real solution is to switch to interference-

resistant modulation techniques, which according to information theory

implies "broadband". Spread spectrum is the purest example, but the

same concepts apply to other systems that are not technically spread

spectrum.

It's well worth reading.

Phil

-------------------------------------------------------------------------------

Date: 10 Nov 1994 22:24:25 GMT

From: karn@unix.ka9q.ampr.org (Phil Karn)

Subject: Pulsed BPSK hoax?(???)

I just came up with an another analysis of K6HH's scheme that shows

that even his bandwidth claim is bogus.

You can model his "narrow shift" BPSK as the sum of an ordinary

antipodal (0/180 degree) BPSK signal plus a much stronger unmodulated

carrier component that is in quadrature (90 degrees) to both BPSK

phases. In his case, the amplitude ("voltage") of the carrier is

approximately 1/.01=100 times that of the BPSK component. That's a

carrier/information power ratio of 10000:1 or 40 dB, with all of that

carrier power going to waste because it doesn't carry any information.

This means a 40 dB power loss vs ordinary BPSK, which is the same

result I got last night.

It's obvious that the bandwidth required by the tiny BPSK signal is

not reduced at all by the addition of the strong carrier component.

It still requires 2 Hz per bit per second, as for standard BPSK. But

look at how the FCC defines "bandwidth" in 97.3(a)(8):

"The width of a frequency band outside of which the mean power of the

total emission is attenuated at least 26dB below the mean power of the

total emission, including allowances for transmitter drift or Doppler

shift."

By this definition, K6HH can claim that his system has *zero*

bandwidth, because all of his sideband energy (the only

information-carrying part) is 40 dB down from his useless

carrier-on-steroids. That puts all of the sidebands outside of the

FCC-defined bandwidth. All that's left is the carrier, which has zero

bandwidth and carries zero information.  The sidebands don't count in

the bandwidth measurement, even though they're still absolutely

necessary to demodulate the signal.

Again, this is a pretty good example of why minimizing transmitter

*power* is so much more important than bandwidth. Not only should the

FCC get rid of bandwidth limits in favor of automatic power control,

there should also be an upper limit on the Eb/N0 that may be required

by any particular modulation technique. This will avoid the gratuitous

interference that could otherwise be caused by using a highly

power-inefficient modulation method and cranking up the wick to

compensate.

Phil

---------------------------------------------------------------------

Date: 10 Nov 1994 21:56:05 GMT

From: karn@unix.ka9q.ampr.org (Phil Karn)

References<CyzAwp.GA4@srgenprp.sr.hp.com> <39sbsd$k0l@qualcomm.com>, <Cz1zKD.802@mv.mv.com>

Subject: Re: Pulsed BPSK hoax?(???)

I've been asked for a more complete citation for the Costas paper.

It's "Poisson, Shannon and the Radio Amateur", and it first appeared

in the December 1959 (!) issue of Proceedings of the I.R.E. It was

reprinted as the first paper in the 1993 IEEE book "Multiple Access

Communications: Foundations for Emerging Technologies", edited by

Norman Abramson, ISBN 0-87942-292-0.

Here's the abstract. --Phil

Summary - Congested band operation as found in the amateur service

presents an interesting problem in analysis which can only be solved

by statistical methods. Consideration is given to the relative merits

of two currently popular modulation techniques, SSB and DSB. It is

found that in spite of the bandwidth economy of SSB this system can

claim no over-all advantage with respect to DSB for this service. It

is further shown that there are definite advantages to the use of very

broadband techniques in the amateur service.

The results obtained from the analysis of the radio amateur service

are significant, for they challenge the intuitively obvious and

universally accepted thesis that congestion in the radio frequency

spectrum can only be relieved by the use of progressively smaller

transmission bandwidths obtained by appropriate coding and modulation

techniques. In order to study the general problem of spectrum

utilization, some basic results of information theory are required.

Some of the significant work of Shannon is reviewed with special

emphasis on his channel capacity formula. It is shown that this famous

formula, in spite of its deep philosophical significance, cannot be

used meaningfully in the analysis and design of practical, present day

communications systems.  A more suitable channel capacity formula is

derived for the practical case.

The analytical results thus obtained are used to show that broadband

techniques have definite merit for both civil and military

applications.  Furthermore, such techniques will result in far more

efficient spectrum utilization in many applications than any practical

narrow-band, frequency- channelized approach. Thus broad-band

techniques can, in many cases, increase the number of available

"channels". With regard to military communications it is shown that

the ability of a communication system to resist jamming varies in

direct proportion to the transmission bandwidth for a given data

rate. Thus narrow-band techniques lead progressively to more expensive

communications equipment and less expensive jammers. It is concluded

that in the military field broad-band techniques are not only

desirable but also often mandatory.

---------------------------------------------------------------------

After the last Dayton Hamvention, W3IWI and I drove to Baltimore and

we discussed at length some ideas I had for a "moonbounce modem". We

eventually came to the conclusion that if my scheme worked via the

moon, it should work even better over HF.

First, some background.

The basic problem is that you're operating over a dispersive, fading

channel. Received phase is essentially random, so coherence is hard to

achieve. But it turns out that you can buy back much of the loss of

noncoherent demodulation -- at the cost of increased bandwidth -- with

m-ary orthogonal signalling.

"Orthogonal" simply means that the signal states are independent of

each other.  You can build a detector that responds to one signal

state while remaining completely unaffected (in theory) by the other

states.  Plain mark/space FSK is 2-ary orthogonal signalling, assuming

the shift is large enough for the signalling rate. You can detect FSK

with two properly tuned filters, and (again in theory) the mark filter

does not respond at all to a space signal, and vice versa.

Ordinary BPSK is not orthogonal, it's antipodal. One state is the

exact opposite of the other, so instead of being independent, they

"fight" each other much like a tug-of-war contest. You use a single

detector and compare the result to a threshold. Antipodal modulation

is 3dB better than orthogonal modulation but it requires phase

coherence, which we don't have on HF.

So we're stuck with orthgonal signalling. But instead of being limited

to only two antipodal signal points, you can have as many orthogonal

signal points as you like. In m-ary orthogonal signalling, you have m

possible channel states, where m is usually a power of 2.  E.g.,

4-tone FSK is a 4-ary orthogonal modulation scheme. Each channel

symbol carries 2 bits of information in the 2^2 = 4 possible states,

assuming only one of the four tones is on at any time (otherwise you'd

just have several 2-ary systems in parallel.)

It turns out that as m approaches infinity, overall performance

asymptotically reaches the theoretical Shannon limit of -1.6dB Eb/N0,

even with noncoherent demodulation! This has been known for a long

time, but unfortunately the improvement with m is too slow to be [practical by itself]..

Given that, here's my idea. Take a reasonable channel bandwidth, say 2

Khz since that will fit through most SSB filters. Divide this 2 Khz

into 256 frequency bins, each 7.8125 Hz wide. In order to keep the

bins independent and orthogonal, our signalling rate will be limited

to no more than 7.8125 symbols per second. But we want a slow symbol

rate anyway to minimize intersymbol interference caused by ionospheric

multipath. Also, synchronization is easier at a low symbol rate; it

may even be possible to use GPS with approximate propagation delay

figures to synchronize "open loop". Anyway, with that many bins we

could send log2(256) = 8 bits per symbol, or 62.5 bits/sec.

Although this scheme by itself is orthogonal, it isn't particularly

resistant to selective fading or narrowband interference. To improve

this I propose another layer of orthogonal coding using Walsh

functions (rows of a Hadamard matrix).

A Hadamard matrix is easily built up recursively. You start with a 1x1

matrix containing the single entry "1". Then you double each dimension

of the matrix by copying it to the right and below unchanged, and

diagonally to the lower right by inverting each bit. Here's the

recurrence relation defined using the limited graphics of ASCII:

M(i+1) = 	M(i)	M(i)

M(i)	~M(i)

You can keep repeating this process until you have a matrix of the

desired size. Here are the first few Hadamard matrices:

1

11

10

1111

1010

1100

1001

11111111

10101010

11001100

10011001

11110000

10100101

11000011

10010110

and so forth.

Now notice that if you take any two rows at random from a Hadamard

matrix and XOR them bitwise, you'll always get an equal number of 1's

and 0's. It's easy to see how this follows from the recursive

definition given above.

This property means that the rows (and the columns, for that matter)

of a Hademard matrix form a set of mutually orthogonal

functions. These are called Walsh functions, and have been around for

a long time. (As an aside, one clever use in the early days of

telephony was to set up wire-crossing patterns for parallel open-wire

transmission lines in order to minimize inductive crosstalk.)

Now let's say that instead of just turning on one tone (out of 256) at

a time for each transmitted symbol, we take the 8-bit symbol and index

the corresponding row from a 256x256 Hadamard matrix. The 256 bits in

that row then control whether or not the tone is turned in each of 256

frequency bins. (The amplitude of each tone is reduced to keep the

transmitter power the same.)

Each symbol still carries only 8 bits, so the energy per bit is the

same in this scheme as with the simple 1-tone-out-of-256

scheme. Moreover, for every Walsh codeword except the all-1s case,

exactly 50% of the bins are on and 50% are off, so the transmitter

power is constant. (The all 1's case could be avoided with some work

if necessary.)

So performance with and without Walsh coding should be exactly the

same over a nonfading gaussian noise channel.  But HF has both

non-gaussian (e.g., narrowband) interference and fading, often of the

frequency-selective kind. Walsh encoding should help by spreading each

symbol's energy out more evenly over the frequency channel.

You could demodulate this signal by first taking a FFT of the raw

input samples to determine the total received energy in each frequency

bin.  Then you take the FHT (Fast Hademard Transform, analogous to the

FFT) to determine which of the 256 Walsh functions was mostly likely

to have been the one that was sent.

Note that the first column in each of the Hademard matrices is a 1,

which is rather redundant.  You could omit it and save a frequency bin

(which is pretty minor). This is called a "simplex" code, as opposed

to a straight orthogonal code. But you could do much better by

augmenting the matrix with each bit inverted, doubling the number of

code words and adding a bit to each symbol. For example, if we take

the 4x4 Hademard matrix

1111

1010

1100

1001

and append to it its bit-inverse, we get

1111

1010

1100

1001

0000

0101

0011

0110

The 8 rows are now either mutually orthogonal (equal number of bit

agreements and disagreements) or antipodal (all bits disagree).

This is called a "biorthogonal" code, and in fact it's one of the

earliest error correcting codes used in spacecraft. Mariner 9 used one

(64x32, I think) at Mars in the late 1960s, before the big switch to

convolutional coding.

The only reason to prefer a simplex or orthogonal code set over a

biorthogonal code is when your channel has a polarity ambiguity (e.g.,

BPSK). You could easily determine polarity with an orthogonal or

simplex code, but not with a biorthogonal code. But we don't have that

problem here, since each "bin" is basically an on-off keyed channel without

any polarity ambiguity.

If we built a 128x128 Hademard matrix and turned it into a

biorthogonal code we could send 8-bit symbols with only 128 frequency

bins. This would let us double our symbol rate to 15.625/sec, or our

raw bit rate to 125 bits/sec. Or we could halve the transmitted

bandwidth and keep the earlier data rate (more on bandwidth issues

later).

Although this scheme should be very robust, there will be bursts of

QRM and deep flat fades too great for it to handle. We could deal with

this by adding another layer of FEC. A natural choice would be a

Reed-Solomon block code over GF(2^8). This code operates on 8-bit

symbols that matche the basic channel symbol. Paul, N9FZX has already

implemented a Reed-Solomon decoder for 8-bit symbols and has made it

freely available over the net. Although the natural block size for a

RS coder over GF(256) is a 255-byte block, you can use a smaller

blocksize if you like by just agreeing between sender and receiver

that the unused symbols are 0s and not sending them. On the other

hand, larger blocksizes are more robust for the same amount of

redundancy.

A more complicated scheme that might work better is convolutional

encoding with soft decision decoding, either Fano or Viterbi.  The

convolutional decoding itself is not a big problem (I've already

implemented and tested both decoders) but what makes it complicated is

the need to modify the Walsh function demodulator to provide soft

samples (instead of "hard" 1s and 0s) for each bit of each demodulated

symbol. There are ways to do this, but they involve a fair amount of

effort. I need to look into this.

I also need to look into the optimality of the biorthogonal

scheme. For maximum efficiency you like to have a channel code where

every possible error event is of equal probability, but the

biorthogonal code doesn't do this. (The probability of one codeword

being turned into its antipodal complement is obviously much less than

the probability of it being turned into another codeword that is

merely orthogonal to it, since the hamming distance between antipodal

codewords is twice that of the distance between two orthogonal

codewords.)  If you can't keep the codewords at equal distance, it

might help to take them into account in your upper layer FEC design.

Or it might not make a big difference, I don't know.

I'm aware that mixing a large number of discrete tones would increase

the variance of the instantaneous transmitter power. My intuition and

the central limit theorem tells me that the instantaneous envelope

power will end up looking very Gaussian. But Shannon says that

efficient modulation schemes *have* to look Gaussian, and it may turn

out that the increased coding gain of this scheme more than makes up

for any increased amplifier losses over a constant-envelope signal.

So in summary, my scheme would take a block of 8-bit data bytes, add

an appropriate number of Reed-Solomon FEC symbols, encode each symbol

into a (128,8) biorthogonal channel code, and then use each of the 128

channel bits to gate a set of 128 discrete channel tones.

Some remarks on bandwidth. The bandwidth efficiency of this scheme may

at first glance appear to be unacceptably low, especially for HF. 125

bits/sec in 2 Khz is only 0.0625 bits/sec/Hz. But the more I learn

about information theory both from the textbooks and from my work here

at Qualcomm, the more I really understand that it's counterproductive

to limit signal bandwidth. In the real world of a shared band, power

efficiency and interference resistance are FAR more important, and

these two properties are possible ONLY with extra bandwidth.

This particular modulation scheme should be incredibly robust against

both noise and QRM. It ought to be easily decodable well below the

operator's ability to even hear it. Because of its spread

spectrum-like properties, strong inband QRM can be notched out fairly

easily. Since the energy distribution of the signal is fairly flat,

after the FFT you just look for any frequency bins with energies way

above average and suppress them before the FHT step. The only limit on

the ability to suppress narrowband QRM (a CW carrier, say) then

becomes the dynamic range of the receiver and A/D converter.

So, what do people think? This stuff represents a radical departure

from the usual approach of trying to minimize congestion by cramming

as much as possible into a narrow HF channel. Because of its wide

bandwidth, we'd need STAs to test my scheme on the HF bands, and a

rule change for general use.  But the FCC's bandwidth rules are based

on intuition that happens to be completely wrong. John Costas said as

much in his classic paper "Poisson, Shannon and the Radio Amateur",

published 35 years ago, but now we actually have the technology to

pursue his suggestions!

Phil

-----------------------------------------------------------------------

>	Phil: a question:  under this scheme, I agree that the power is

>constant, however it would seem that the peak amplitude could be very

>high.  Would this require a great deal of linearity in the transmitter

>and receiver?

Yes, it would. I address this particular point later on. I assert (without

proof) that the coding gains would make up for any losses due to having

to back off your amplifier to keep it linear.

Phil

-----------------------------------------------------------------------

On Tue, 15 Nov 1994, Phil Karn wrote:

>

> Some remarks on bandwidth. The bandwidth efficiency of this scheme may

> at first glance appear to be unacceptably low, especially for HF. 125

> bits/sec in 2 Khz is only 0.0625 bits/sec/Hz. But the more I learn

> about information theory both from the textbooks and from my work here

> at Qualcomm, the more I really understand that it's counterproductive

> to limit signal bandwidth. In the real world of a shared band, power

> efficiency and interference resistance are FAR more important, and

> these two properties are possible ONLY with extra bandwidth.

>

Hmmm...

My point of reference is the work I've been doing here at Motorola. We

build RF modems which operate at 19.2 kbps over 25 kHz channels for an

efficiency of 0.768 bits/sec/Hz. The modulation scheme is 4-tone FSK with

Trellis coding and forward error correction. Of course, the Trellis coding

is overhead which reduces the effective bit rate. Does the larger

bandwidth that we have available allow us greater efficiency than that of

your proposed scheme? Do we approach some sort of limit (i.e. Shannon's)

as the system becomes more spread-spectrum like? BTW, our products operate

at VHF so we aren't too concerned about dispersion. However, we do have to

deal with fading as they were originally designed for mobile applications,

although they're finding their way into portable applications these days.

>

> So, what do people think? This stuff represents a radical departure

> from the usual approach of trying to minimize congestion by cramming

> as much as possible into a narrow HF channel. Because of its wide

> bandwidth, we'd need STAs to test my scheme on the HF bands, and a

> rule change for general use.  But the FCC's bandwidth rules are based

> on intuition that happens to be completely wrong. John Costas said as

> much in his classic paper "Poisson, Shannon and the Radio Amateur",

> published 35 years ago, but now we actually have the technology to

> pursue his suggestions!

>

I'm trying to hoist it all in! I'm somewhat of a newbie to HF data so I

don't have too many preconceptions. If a wider channel allows greater

efficiency and better immunity to QRM then it's hard to argue against

this on strictly technical grounds. Of course there are politics &;)

I haven't played with the sound card much yet. Is it fast enough to

perform the transforms that you mention? Maybe benchmarking these

algorithms would be a good place for me to get started? What do you say

Johan?

Hugh

------------------------------------------------------------------------

Hi,

After just briefly browsing through Phil's post - my $0.02.

It is true that for practical reasons, non-coherent techniques makes a lot

of sense. I just wonder though, whether we should totally disregard the

possibility of extracting phase information with some reliability over HF

links - this factor will play a key role of how we should trade off

spectral efficiency and bandwidth efficiency. My question is: do we know

enough to say yes or no at this point in time? I suspect we don't.

If we judge from the experiences with of commercial MIL-STD-188 HF modems,

we see that they do implement some of the things that Phil mentioned, i.e.

orthogonal signalling in the form of 16 or 39 parallel carriers, however,

each being phase modulated. I suspect that perhaps some tradeoff is

possible and that is, IMHO, where we can made our contributions.

Coding theory could do wonders when done right and perhaps should be part

of the modulation as Phil suggest, however, I hope we do justice to

modulation basics. I hope the simulator would help in providing some clues.

If I had my druthers, I would have opted for SS, but that is perhaps

looking too far into the future. Or is it?

73's

Johan, KC7WW

P.S. pse e-mail me at: forrerj@ucs.orst.edu

-----------------------------------------------------------------------

The current issue of QEX has a rather interesting article on the

results of some tests on Clover II using an HF channel simulator.

The character error rate vs Eb/N0 curves (no ARQ) are very

telling. They compare several Clover data rates against a NATO HF

modem standard, STANAG 4285. What's particularly interesting is that

in both fading situations (1 and 2-path Rayleigh) STANAG 4285

outperformed CloverII by 10 dB or more for the same data rate (300

bps).

The author attributes much of the difference to STANAG 4285's use of a

3 Khz channel vs Clover's use of a 500 Hz channel. I think this

illustrates very nicely the perils of narrowband thinking. Consider

that STANAG 4285 not only requires less than 1/10 of the total

transmitter power of Clover II for the same data rate, but it spreads

it out over 6 times the bandwidth. This reduces the signal spectral

density by more than 60x or 16 dB. That's a significant reduction in

interference potential to others reusing the channel elsewhere.

The article also mentions MIL-STD-110A, saying that it's very similar

to (but incompatible with) STANAG 4285. Since we have a CD-ROM

database of MIL-STDs at work, I was able to print off a copy (81

pages). If someone is really interested in a copy, I could be

persuaded to make some more and mail them out.

Phil

------------------------------------------------------------------------

>        With little or no multi-path signal distortion, 300 baud, and

>        above is possible.  However, in most cases you must be below

>        150-200 baud to prevent this problem from causing detection

>        problems.  In severe cases, the problem may exist down to 100

>        baud.

So don't use a high baud rate. This implies multiple bits per

transmitted symbol (channel state change). This can be done either

orthogonally or non-orthogonally. Orthogonal simply means that all of

the possible channel states can be detected independently, without

mutual interference. Binary FSK with a sufficiently wide shift for the

baud rate is the classic example; you can build a "tone filter" that

responds just to mark, and another one that responds only to space.

But nothing limits FSK to just two tones. If you used, say 256

distinct tones then you could encode the 256 possible combinations of

an 8-bit data symbol as one of the 256 tones. This would be 256-ary

FSK.  The general scheme is called m-ary FSK and has long been known

to be one of the best schemes around for fading HF channels. The

higher the value of m, the better the performance -- assuming you can

spare the bandwidth.

The most common NON-orthogonal signalling scheme for multiple bits per

transmitted symbol is QAM. This maps a n-bit data symbol into 2^n

possible combinations of carrier amplitude and phase.  QAM's big

advantage is very good bandwidth efficiency for large values of n,

which is why it's universal in telephone line modems. But big values

of n mean very closely packed signal constellations, and relatively

little noise is required to move one received signal state to another

one nearby and cause an error. So the bandwidth efficiency of QAM

comes at the price of a much higher required S/N ratio. This is again

okay for telephone modems where the S/N ratio is usually 30 dB or

more, but (in my opinion) it is completely inappropriate for noisy,

interference-prone radio channels.

So once again we're back to the conclusion that to build a decent HF

modem that provides a good link with minimum transmitter power, we

really need to do something to relax the bandwidth limits.

Phil

------------------------------------------------------------------------

Phil Karn wrote (12/20/94):

>So don't use a high baud rate. This implies multiple bits per

>transmitted symbol (channel state change). This can be done either

>orthogonally or non-orthogonally. Orthogonal simply means that all of

>the possible channel states can be detected independently, without

>mutual interference. Binary FSK with a sufficiently wide shift for the

>baud rate is the classic example; you can build a "tone filter" that

>responds just to mark, and another one that responds only to space.

>But nothing limits FSK to just two tones. If you used, say 256

>distinct tones then you could encode the 256 possible combinations of

>an 8-bit data symbol as one of the 256 tones. This would be 256-ary

>FSK.  The general scheme is called m-ary FSK and has long been known

>to be one of the best schemes around for fading HF channels. The

>higher the value of m, the better the performance -- assuming you can

>spare the bandwidth.

As supporting arguments to above, please see J.D. Ralphs's book:

Principles and practice of multi-frequency telegraphy. Peter Peregrinus,

ISBN 0-86341-022-7. This book includes extensive chapters on

how such a system is engineered around the symbol set and signaling rate to

combat the properties of the HF channel. It also compares this m-FSK to the

MIL-STD-188A 16-tone parallel modems and comes out a nose ahead of the

pack. The book also describes a simple synchronous detector that would

be quite simple to implement on our present DSP platforms. If you are

interested in using an HF channel simultor this reference also contains

very useful stuff.

----------------------------------------------------------------------

>The possibilities of QAM is quite attractive. Its signaling

>constallation is shown to have distinct robustness over other

>alternative schemes of phase/amplitude signaling especially under

>conditions that include conditions of phase jitter (please see the Foschini

>paper that I posted a while ago).

I'm not convinced. QAM requires phase-coherent detection, but the HF

channel is inherently phase random. A Rayleigh fading channel model

has a Rayleigh probability distribution on amplitude (hence the name)

and a uniform distribution of phase between 0 and 2pi - i.e., the

received phase is completely random and can carry no information due

to the constantly changing and unpredictable propagation path lengths.

Although you might be able to get away with phase modulation on some

particularly benign HF channel, especially if there are only a small

number of path components, it sure doesn't seem like a wise thing to

rely on in your design.

So this implies noncoherent demodulation, i.e., just looking at signal

energy and ignoring phase. m-ary FSK can be demodulated this way. As

you increase m towards infinity you not only gain back everything you

lost by going noncoherent, but you also approach the -1.6dB Shannon

limit!

This stuff actually works. Although the details and implementation are

quite different, Qualcomm's CDMA system uses 64-ary modulation on the

reverse (mobile-to-cell) link in order to make up for the lack of a

coherent carrier phase reference over the Rayleigh-faded mobile radio

channel. The orthogonal codes are Walsh functions over time instead of

collections of sine waves over frequency, but the same principles

apply.

--Phil

------------------------------------------------------------------------------------

I haven't posted to a list before, so let's see if this works...

In all the discussion about the best modulation format for HF, I'm

surprised nobody has mentioned OFDM.  Orthogonal frequency-division

multiplex is proposed for Digital Audio Broadcasting and digital TV

in Europe, although it is not so well known in the US.

The idea is to split the signal up into a large number (like 128 or

1024) equally-spaced carriers, each modulated at the same symbol rate.

I believe the DAB standard proposes that each carrier be modulated

with QPSK, but you can use any modulation in which each symbol can be

represented by a phase and amplitude.  That includes the many forms of

QAM, PSK, PI/4 DQPSK, on-off keying, whatever.

For example, If you use 128 carriers, the symbol rate is reduced by

a factor of 128, greatly reducing the probability of inter-symbol

interference due to multipath propagation.  Adding forward error

correction and time and frequency interleaving reduces the probability

of bit errors due to selective fading on one or more carriers.

The reason the mode is called "orthogonal" FDM is that the carrier

frequency spacing is 1/symbol period, which theoretically eliminates

interference from all unwanted carriers.  This is easiest to see in

the time domain.  For a symbol rate of 1 symbol/s, the carriers will

be spaced 1 Hz apart.  During each symbol period, the detector for

each carrier receives a certain number of RF cycles of the desired

carrier.  During that same time period, each unwanted carrier has a

number of cycles that differ from the desired carrier by an integer.

Convolving two sine waves that differ by an integer number of cycles

gives zero result.

You could implement OFDM by building 128 RF oscillators, 128 I/Q

modulators, and the associated controlling hardware.  But there's a

much simpler way, and here's the clever part of OFDM:

I mentioned that the signal consists of a series of equally-spaced

RF signals, each characterized by an amplitude and a phase.  If you

do an inverse DFT on that data, you come out with the time samples

of the composite signal!  At the kinds of data rates we are talking

about, an inexpensive DSP would be plenty fast enough.  For example,

a 12.5 MHz ADSP21xx can do a 256-point DFT in 0.59 ms.  So we're

talking about a $10 DSP and a single I/Q modulator running at very

low symbol rates.

For example, 128 QPSK carriers spaced 7.5 Hz apart (7.5 baud) gives

1920 bits per second within the 1 kHz bandwidth limit.  The DSP

has 1/7.5 = 133 ns to do all its calculations.

The choice of modulation type for the carriers affects only the

amplitude/phase list for each symbol.  So it would be easy to

change modulation types to respond to different propagation

conditions, ala Clover.  By including the capability to do various

size FFTs, different symbol rates could be implemented using the

same bit rate within the same bandwidth.  i.e.  use very slow

symbol rates (large FFT) under fast Rayleigh fading conditions

and faster symbol rates (small FFT) with slow flat fading.

Alan Bloom N1AL

--------------------------------------------------------------------

[12-21-94]

Some further food for thought:

Here are some further details on the 16-tone MIL-STD-188C modem

and a comparitive M-FSK system, just for interest sake.

This MIL-STD modem uses 16 simultaneous tones, each modulated at 75 baud

(13.3 ms elements) in two or four phases giving a maximal rate of

2x75x16=2400 bit/s in 3 kHz. The tones are spaced at 110 Hz, orthoganal

for the matched filter integration period of 9.1 ms, which allows for a

guard time of +/- 2ms to cater for a limited degree of sych. error.

The provision of guard periods causes an effective power loss of

1.75 dB. Assuming each tone quadrature modulated, the normalized bandwidth

is 3 Hz/bit/s.  However, extending this to 16 parallel tones, this yields

0.87 Hz/bit/s, which is pretty close to the ultimate.

I have some further specifications on its BER, but unfortunately not

with me at the moment. Walt, K5YFW, could perhaps attest how these

have been doing on the military HF nets. This is living proof that

phase modulation is worth considering on HF.

One consideration when dealing with such a multi-tone signaling system,

that the power rating of the transmitter is rated on the assumption that a

single unmodulated carrier is radiated, however, with multiple tones

the output power is reduced by the square of the number of tones. Also

remember that, when dealing with the combination of such multiple tones

that a "noise-like" waveform results with possibly high peak/RMS ratio

(i.e. crest factor) that may cause distortion or inter-modulation effects.

According to Ralph's book, a 6-channel 6-FSK (i.e. six 6-FSK systems

paralleled using similar bandwidth) outperforms this MIL-STD-188C

modem. Sounds interesting?

73's

Johan, KC7WW

------------------------------------------------------------------------

[12-21-94]

Gee Alan, kinda sounds like the 16 & 39 parallel tone modem formats in

MIL-STD-188-110c.  16 or 39 parallel tones, with QPSK, inter-leaving and

Reed-Soloman coding for FEC.  In the 39 tone modem, a 40th tone has no

modulation and is called the Doppler Tone.  If phase shift is detected

on it, the modem knows than inter-symbol interference due to multipath

propagation is taking place.  The baud rate on the 39 tone modem is less

than 50 baud and the bandwidth of the signal is 3 KHz.  The transmitted

"data thruput rate (their terminology) is 2400 BPS.  The actual

transmitted bit rate is 3200-3600...I've forgotten and don't want to dig

into my files tonight.  The CLOVER II is a 4 parallel tone modem (and a

few other things) and runs 850-900 BPS.

Walt@home.2.nite

----------------------------------------------------------------------

[12-21-94]

Alan Bloom, N1AL wrote:

>On the receiving end, I presume you would just run a DFT on the

>incoming sampled data which would output the frequency and phase

>of each carrier.  It is effectively coherent detection, since

>the phase is known.

That is good news Alan. After your post, I spent last night digging out

all the stuff that I had on the MIL-STD-188 39-tone parallel modems, and

what a surprize - now that I know a little better what to look for.

The 39-tone system is, by definition, what you called ODFM. It comprizes

39 tones, spaced to obtain orthogonal signaling, with each tone TDPSK

(4 phase) modulated (I don't quite understand yet what this means? Anyone

pse). Although very few manufacturers actually tell you how they do

their demodulation, I found one reference to it: 128 point FFT! All these

systems occupy a typical voice bandwidth channel and even when it operates

at lower bit rates, never uses less bandwidth, but uses redundancy instead.

The figures for BER for these 39-tone modems, as specified minimum

performance specs (from MIL-STD-188-110 document), requires:

1.8e^-4 at 30 dB S/N for 2400 bps, 2.7e^-6 also at 30 dB S/N for 1200 bps.

At 75 bps 1.0e^-6 for 8 dB S/N. I think wwill agree that this is not

bad. Keep in mind that all these BER merits relies heavily on coding,

i.e. block or convolutional combined with interleaving. All the specs that

I looked at also includes some form of tracking for doppler, i.e. +/- 75Hz,

at a rate of 3.5 Hz/second. One small tidbit was that one manufacturer

actually disclosed that an M68030 was used as a controller in conjunction

with two M56002's.

Does anyone have further references to such ODFM modulation schemes?

I would be very interested to hear more.

Thanks for all the excellent contributions - we are making good

progress.

73's

Johan, KC7WW

------------------------------------------------------------------------

[12-21-94]

Walt@home.2.nite said:

>Gee Alan, kinda sounds like the 16 and 39 parallel tone modem formats in

>MIS-STD-188-110c.    ...

Kinda.  Although with only 16 or 39 tones, using a FFT/IFFT is not very

efficient, since you don't have 2^N tones.  With fewer tones, you need

a higher symbol rate to get a desired bit rate, which would require

more DSP horsepower.  As Bob said:

BobH said:

>If we don't have the MIPS and the algorithms for the STANAG and the

>MIL-STD adaptively equalized single tone hf modems...

The advantage of OFDM is the computational efficiency due to using the

FFT.  Equalization is easy:  Just adjust the amplitude and phase of

each of the carriers in the frequency domain before doing the IDFT.

Larry Gadallah said:

>I have also considered the idea that OFDM is a coarse grained form of

>spread spectrum, albeit with little spreading.  ...

It's not really spread, since the total bandwidth is about the same

as a single carrier modulated with the same bit rate.

By the way, on the subject of bandwidth, there is no need for

pre-modulation filtering of the transmitted signal (other than the

standard DAC anti-aliasing filter).  That's because each carrier has

such a low baud rate that even with unfiltered square waveforms

the sinx/x frequency rolloff is fast enough to give an almost-square

frequency spectrum for the composite signal.

Alan

------------------------------------------------------------------------------------

[12-22-94]

Phil,

Thanks much for the brief note on Shannon - it always is a

good reality check to keep those principles in mind.

OK - I am ready to start playing with some of these great ideas - perhaps

we can start looking into some of the practical details now.

It would not be difficult to simulate the multi-tone signals in a

C program, and then play with the different ideas on demodulation.

It appears there are two ideas put forward:

1) (ODFM) n-parallel carriers with orthoganal signaling. All carriers

are always present. Demodulation is performed by FFT's. It is obvious that

the choice of the FFT size and sample rate have to just right otherwise we

will end up with an odd distribution of our carriers over the

frequency/phase bins. After this choice have been made, how do I use this

I/Q recovered data? I have some wild ideas, but would like to obtain

further input first.

2) (Phil's method based on Welch functions) n-parallel tones with orthoganal

signaling. Only a small subset of carriers are present at a time - the

different subset is selected from a Hamadard matrix. In a sense,

Piccolo may be considered a special case of this idea, where only one

carrier is sent at a time from a set of m-carriers. Phil mentioned that FHT

(is that for fast hadamard transform?) is used to perform demodulation. How

about telling us a bit more about how to implement this one Phil.

73's

Johan, KC7WW

-----------------------------------------------------------------------

[1-3-95]

Hi All,

Trust you had a good holiday season - best wishes for 1995.

Apparently there were some problems with the listserver - hope it is

working now.

Bob asked an interesting question: whether soft decision decoding

with convolutional coding was as effective as linear block coding

with interleaving. There is an interesting, though rather oldish, paper

about this that you may find interesting:

"Effective Application of Forward-Acting Error-Control Coding to

Multichannel HF Data Modems" by Pierce et. al. IEEE Trans. on COMM,

Vol COM-18 (4), August 1970. pp 281-294.

This paper compares early multi-tone parallel modems over HF paths and looks

at how different phase and frequency shift modulation formats perform in

conjunction with various linear block, as well as, convolutional codes. It

appears that some block codes (i.e. (23,12)) code with interleaving does as

well as a convolutional code with the appropiate constraint length (very

large), however, it is not a simple matter to engineer the coding scheme

without regard to the modulation scheme - these things should be engineered

to work together and not separate. This comparison was by live

experiments over transcontinental as well as transatlantic HF paths, and is

about as old as the work done by Watterson on the HF channel simulator.

I still am looking for further information on the Fast Hadamard Transform

(or the Fast Walsh Transform). If anyone can shed some light on this, it

would be greatly appreciated.

73's

Johan Forrer, KC7WW

------------------------------------------------------------------------

[1-9-95]

I have been following the discussion on OFDM and studying

Phil's intriguing proposal for using M-ary orthogonal FSK

with Walsh functions.  Over Christmas break (I'm pursuing

an MSEE; if only I could catch it) I did some crude

simulations of Phil's scheme using MATLAB.

First, I generated a 64x64 Hadamard matrix (whose rows are

Walsh functions).  Then I generated 64 equal amplitude

sinusoids, each separated by 32 Hz, from 512 Hz to 2528 Hz.

I then picked a row at random from the Hadamard matrix to

determine which sinusoids to sum together.  I sampled the

time sequence at 32 kHz and plotted it over a full period

of the combined signal.

I used several different Walsh functions to get a feel for

the general behavior of these signals.  Since I started with

64 tones, and only 1/2 are on for any particular Walsh

function, I was combining 32 separate tones.  The results

were rather interesting, and very similar to Johan's.

Thus wrote Johan, KC7WW:

> As a way to learn more about how OFDM works for the

> 39-tone MIL-STD-188 parallel modem, I wrote a C program

> that simulates 39 parallel tones and combines them into

> a single output signal.

[snip]

> I also did capture the simulated output waveform and was

> quite concerned after inspecting the plot  -  the

> presence of large spikes of significant magnitude (in

> some cases, some 10-15 dB over the RMS value).

My simulation produced signals with peak-to-RMS ratios of

9 dB, they were generally of low amplitude except where

the sinusoids came into phase and produced a large peak,

and they generally looked 'noiselike'.  I also noticed that

the signal's peak amplitude was not symetric about the time

axis.  The signals had zero average value but the largest

positive-going peak was twice as high as the largest

negative-going peak.

As several others have noted, this high peak-to-RMS ratio

will limit the amount of energy per symbol.  A signal such

as this is potential trouble in any amplifier unless care is

taken to control the peak level.  The asymetry in peak

heights may furthur limit the amount of power an amplifier

could deliver.  This is definitely not a constant envelope

signal so linear amplifiers are in order.  As more tones are

added the signals will have larger peak-to-RMS ratios.  This

is consistent with Johan's observations since he was summing

more tones.

The FCC limits amateurs based on PEAK envelope power (PEP),

not RMS power.  As Phil noted, coding gains can buy back

most of the loss in energy per symbol caused by the low RMS

value of the signal, but it's still a significant limitation.

Disregarding interference, it's ultimately the energy per

bit that determines the performance of a digital system.

Phil's proposal was for a noncoherent system, which got me

thinking (scary, eh?).  A noncoherent system ignores the

phase of a signal; so, we have a free parameter to play with.

My initial simulation started each sinusoid at zero phase.

If we could use the phase to reduce the peak value of the

signal we could get more energy per symbol and get the coding

gain as well.

First I tried a linear shift in the initial phase across all

64 sinusoids.  I started with zero initial phase at the lowest

frequency and increased the phase linearly to two pi at the

highest frequency.  This resulted in signals with about the

same peak-to-RMS ratios (8.25 dB), still 'noiselike', but the

amplitude peaks became symetrical about the time axis

(interesting and unexpected).

My next attempt was to assign a random initial phase to

each of the 64 sinusoids. The initial phase was distributed

uniformly between zero and two pi.  This produced signals with

an average peak-to-RMS ratio of 4.75 dB, the signals looked

extremely 'noiselike', and the amplitude peaks were nearly

symetrical.  Progress!

These were just the first ways I thought of to change the

phase, there was no method to my maddness.  Even a naive

method like ramdomizing the phase produced a 4 dB reduction

in the peak-to-RMS ratio.  These results lead to a conjecture

and several questions.

My conjecture is that we can reduce the peak-to-RMS ratio

further, and maybe even approximate a constant envelope

signal, by properly choosing the phases of the sinusoids.

I think this is plausible since a wideband FM signal is a

constant envelope signal and its spectrum looks like a set

of nearly equal amplitude sinusoids, within a limited

bandwidth.  I doubt we could get a truly constant envelope

signal because we are using a small set of harmonically

related sinusoids.

Some questions I have:

-  Are there systematic methods for choosing the

initial phases of the sinusoids to minimize the

peak-to-RMS ratio?

-  What limits are there on reducing the peak-to-RMS

ratio?

-  Are these limits, if they exist, related to the

number of sinusoids or the Walsh function used?

-  Is this a well-known (or obscure) problem that

already has a solution?

-  Is reducing the peak-to-RMS ratio really as

important as I seem to think it is?

The MIL-STD modems apparently use PSK on the tones as well.

Since the phase of any particular sinusoid in my simulation

was constant over the symbol period this should allow using

methods such as DPSK on the tones if someone wants to add an

additional layer of modulation.  Would this increase the

peak-to-RMS ratio?

I can provide the MATLAB M-files I used for this simulation

if anyone is interested in duplicating, or extending, my

results.  Thoughts anyone?

Eric, N2NNP

---------------------------------------------------------------------

[1-9-94]

Eric E. Silbaugh wrote:

>I have been following the discussion on OFDM and studying

>Phil's intriguing proposal for using M-ary orthogonal FSK

>ith Walsh functions.  Over Christmas break (I'm pursuing

>an MSEE; if only I could catch it) I did some crude

>simulations of Phil's scheme using MATLAB.

>First, I generated a 64x64 Hadamard matrix (whose rows are

>Walsh functions).  Then I generated 64 equal amplitude

>sinusoids, each separated by 32 Hz, from 512 Hz to 2528 Hz.

>I then picked a row at random from the Hadamard matrix to

>determine which sinusoids to sum together.  I sampled the

>time sequence at 32 kHz and plotted it over a full period

>of the combined signal.

.....snip.....

>   -  Are there systematic methods for choosing the

>      initial phases of the sinusoids to minimize the

>      peak-to-RMS ratio?

Eric, just a suggestion: you may want to look at an earlier (way back)

message thread on this very topic. It is recorded in an archive on ftp.ucsd

in the DSP directory. If you have a problem obtaining it, I'll dig out the

name of the archive.

I think there is no good news here. There is no easy solution as far as

I can gather (?). CLOVER, as an example, addressed this problem by using

time and frequency interleaved signaling as well as pulse shaping.

..... more snipping .....

>The MIL-STD modems apparently use PSK on the tones as well.

>Since the phase of any particular sinusoid in my simulation

>was constant over the symbol period this should allow using

>methods such as DPSK on the tones if someone wants to add an

>additional layer of modulation.  Would this increase the

>peak-to-RMS ratio?

I suspect it depends on the peak of the individual channel waveform.

>I can provide the MATLAB M-files I used for this simulation

>if anyone is interested in duplicating, or extending, my

>results.  Thoughts anyone?

I, for one, am quite interested in your work, Eric. Likewise, if

anyone is interested in the OFDM simulation, I would be happy to share

the "C" source code.

A bit of progress over the last weekend: I finished a FFT-based display

scope program for the DSP sound card. Since I am interested in using the

FFT-approach for doing an OFDM modem, thought that I could use the same

FFT for different purposes. It's a 256-point radix-4 that includes

a Hanning window (and log transform for calculations in dB, if needed) and

does it in approximately 250 micro seconds. The first thing I did was to

look at the audio frequency responses of my ICOM 751 - boy! was I

surprised.

I went ahead and played with the DSP sound card and implemented the parallel

tone modulator and FFT-based OFDM demodulator - it is surprising to see

how sensitive the FFT is to any clipping effects of these large spikes in

the signal. One has to be real careful of this, else a bit of garbage seem

to appear in the wrong frequency bins. However, when this kind of garbage

ends up in the wrong frequency bin, it seems to appear as uncorrelated

noise - the actual signal in the frequency bin still appears quite distinct

and recoverable. Perhaps this side-effect is not all that serious

after all. Comments?

73's

Johan, KC7WW

-----------------------------------------------------------------------

[1-11-95]

Johan says:

>Bob asked an interesting question: whether soft decision decoding

>with convolutional coding was as effective as linear block coding

>with interleaving.

I'd been meaning to comment on this, glad you reminded me.

I've learned that this has been a rather controversial subject that

some people argue with a fervor that approaches the classic VC vs

datagram wars of 10 years ago.

Up front I should say that I happen to work for one of the main

parties to this debate (Andrew Viterbi) so my comments may be biased.

Convolutional coding has the following general advantages:

1. Better performance for equivalent code complexity

2. Can use "soft decision" information for improved performance

(2dB on AWGN) much more easily

3. More easily adapted to arbitrary block sizes

Block codes, as typified by Reed Solomon codes, have the following

general advantages:

1. No performance penalty for "systematic" codes (where the data

appears unchanged as a subset of the coded output)

2. Complex codes more easily implemented (partially negating

convolutional advantage #1)

3. No need to "tail off" blocks for full coding gain

4. Less sensitive to burst errors

5. Direct adaptability to higher-order modulation symbol alphabets,

such as the m-ary FSK schemes I've been discussing.

Nobody really disputes the correctness of any of these points, since

they're amenable to mathematical proof. The arguments all center over

the relevance of their assumptions, and which advantages are more

important in practice.  For example, both Viterbi and Reed-Solomon

decoders are amenable to VLSI implementation, but differences in the

details can give one a temporary advantage over the other.

It's also common to combine convolutional and block codes to get the

best of both worlds. These "concatenated" coding systems are commonly

used in deep space communications, starting with Voyager. You run

a Reed-Solomon block code on top of a Viterbi-decoded convolutional

code, usually with an interleaver between the two.

The Viterbi decoder does most of the work, and since it's directly

connected to the modem it can benefit from soft decision samples. But

when the Viterbi decoder is temporarily overwhelmed, it emits bursts

of "hard" (binary) errors that are well adapted to the capabilities of

the Reed Solomon decoder.

The Voyager concatenated coding system is fairly simple by modern

standards and is well within our capability. I've already done the

Viterbi decoder for the NASA standard r=1/2 K=7 convolutional code.

N9FZX has done a Reed-Solomon decoder that may be the same one that

Voyager uses, but I'm not sure. In any event, all you need to combine

the two is some glue (interleaving, buffering, etc). The waterfall

curve for the Voyager coding scheme is a nearly vertical wall at

around 2-2.5dB.

Galileo uses the same basic techniques, but taken to some wild

extremes to squeeze the most out of the omnidirectional antenna since

the failure of the main antenna. The convolutional code has r=1/4,

K=14, which is extremely high for a Viterbi decoder (the work factor

increases exponentially with K). And the RS code uses varying amounts

of redundancy per block which permits a trick called "redecoding".

First you run the Viterbi decoder. Then you run the RS decoder on the

output.  Some of the RS blocks will decode, some won't; the blocks

with more redundancy being more likely to decode. You fill in the

missing symbols in the decoded blocks and then toss them back to the

Viterbi decoder for another go with the RS-decoded states "pinned

down". Then you have a second go at the RS decoder. You iterate this

perhaps 4-5 times until things stop improving. The end result is an

astonishingly low Eb/N0 requirement of about 0.56dB, damn near the

Shannon limit for that channel. Of course, they do this at only 10 b/s

on a Sparcstation...

Phil [Karn]

-----------------------------------------------------------------------

[1-11-95]

Hi Phil,

That essay on coding was welcomed by everyone - well done!

I would just like to add, if I may, one further thought relating

specifically to the multitone signaling systems and coding methods. In a

previous posting I mentioned the work by Pierce et, al. - nobody seems to

have commented on that. Before it is lost - here is a brief summary:

The fact that was established in this research paper, that was based on

actual transatlantic and transcontinental HF data links at various HF

frequencies, was that HF disturbances has somewhat characteristic

nature, either broadband and/or noise bursts in conjunction with various

forms of fading. This often has some frequency/sequency effects that could

be put to good use in the type of coding applied, i.e. the choice of

interleaving/constraint length for example.

The second fact that the research established, was that channel spacing,

(i.e. signaling over othoganally spaced channels) appears to have a

systematic effect on BER. For example, if you look at BER vs. coding

scheme, cyclic behavior with distinct minima is observed - that tells us

that it is possible to optimise coding gain by proper choice of

parameters, such as constraint length, interleave factor, and type of code.

Taking these observations into account, as I have indicated in my earlier

posting - an appropiate convolutional coder or the right type of block

code was equally effective on parallel tone modems. This reseach was done

prior to soft-decision coding becoming popular and I am sure other advances

were made in coding theory since.

There is no question that a combined block/convolutional code, possibly

with soft-decision coding, will buy us the best of both worlds. Coding

theory, however, is not one of the easiest of fields to wrestle with - for

one, it does not allow the same degree of intuition or gut feeling to be

applied as for instance modulation/demodulation theories. You really have

to know what you are doing.

I think we all agree that we are very much dependant on the guidance and

contribuitions of experts like Phil.

Looking forward to learning more about this field.

73's

Johan, KC7WW

-----------------------------------------------------------------------

[1-12-95]

Here's an abstract I found on a NASA web server that summarizes the

coding tricks they're using to milk the most out of the low-gain

antenna on Galileo. Of course, the deep space channel is very

different from the HF channel, but it gives you an idea of what coding

can do.

Enhanced decoding for the Galileo S-band mission

DOLINAR, S.; BELONGIE, M.

Jet Propulsion Lab., California Inst. of Tech., Pasadena, CA.

Published: August 1993           Pages:  00016       Biblio Refs: 0

A coding system under consideration for the Galileo S-band low-gain

antenna mission is a concatenated system using a variable redundancy

Reed-Solomon outer code and a (14,1/4) convolutional inner code. The

8-bit Reed-Solomon symbols are interleaved to depth 8, and the eight

255-symbol codewords in each interleaved block have redundancies 64,

20, 20, 20, 64, 20, 20, and 20, respectively (or equivalently, the

codewords have 191, 235, 235, 235, 191, 235, 235, and 235 8-bit

information symbols, respectively). This concatenated code is to be

decoded by an enhanced decoder that utilizes a maximum likelihood

(Viterbi) convolutional decoder; a Reed Solomon decoder capable of

processing erasures; an algorithm for declaring erasures in undecoded

codewords based on known erroneous symbols in neighboring decodable

words; a second Viterbi decoding operation (redecoding) constrained

to follow only paths consistent with the known symbols from

previously decodable Reed-Solomon codewords; and a second Reed-

Solomon decoding operation using the output from the Viterbi

redecoder and additional erasure declarations to the extent possible.

It is estimated that this code and decoder can achieve a decoded bit

error rate of 1 x 10(exp 7) at a concatenated code signal-to-noise

ratio of 0.76 dB. By comparison, a threshold of 1.17 dB is required

for a baseline coding system consisting of the same (14,1/4)

convolutional code, a (255,223) Reed-Solomon code with constant

redundancy 32 also interleaved to depth 8, a one-pass Viterbi

decoder, and a Reed Solomon decoder incapable of declaring or

utilizing erasures. The relative gain of the enhanced system is thus

0.41 dB. It is predicted from analysis based on an assumption of

infinite interleaving that the coding gain could be further improved

by approximately 0.2 dB if four stages of Viterbi decoding and four

levels of Reed-Solomon redundancy are permitted. Confirmation of this

effect and specification of the optimum four-level redundancy profile

for depth-8 interleaving is currently being done. Author (revised)

NASA Subject Category: COMMUNICATIONS AND RADAR

Major Subject Terms:   CONCATENATED CODES, DECODING, GALILEO PROJECT,

MAXIMUM LIKELIHOOD ESTIMATES, MICROWAVE ANTENNAS,

REED-SOLOMON CODES

Minor Subject Terms:   BIT ERROR RATE, CODING, DECODERS, SIGNAL TO NOISE

RATIOS, SUPERHIGH FREQUENCIES, SYMBOLS, ULTRAHIGH

FREQUENCIES

CASI Accession Number: N941438

Contract Number:       RTOP 314-30-61-

Report Number:         none

Translation Note:      none

Ordering Information:  Avail: CASI HC A03/MF A03

Document Class:        unknown

------------------------------------------------------------------------

[1-16-95]

>Taking these observations into account, as I have indicated in my earlier

>posting - an appropiate convolutional coder or the right type of block

>code was equally effective on parallel tone modems. This reseach was done

>prior to soft-decision coding becoming popular and I am sure other advances

>were made in coding theory since.

Soft-decision decoding wins especially big on fading channels, by the

way.  As I mentioned in my discussion, convolutional decoding is much

more easily adapted to soft decision decoding. It's almost universal

in Viterbi decoding, somewhat less so in sequential because of its

greater sensitivity to gain errors.

Soft decision decoding can be adapted to block codes, but with

considerably greater difficulty. The technique is called "Chasing

bits", after David Chase, the inventor. Basically, you determine your

N weakest received symbols. Then you try decoding all 2^N possible

values of these weak symbols (for a binary code) along with the

"strong" received symbols. Take each decoded block, re-encode it and

pick the result that's closest in Hamming distance to the original

received block of symbols.

Obviously the complexity goes up exponentially with N. That can really

stress your block decoding software.

Non-binary codes like Reed Solomon codes can be "softened" somewhat by

simply erasing your weakest symbols. Since a RS code can correct twice

as many symbol erasures as errors, you stand an improved chance of

decoding the block correctly.

Erasure decoding on a binary channel can be seen as soft-decision

decoding with only 3 symbol values: "1", "0" and "unknown". A little

"softness" goes a long way; just adding an erasure indication is a big

win, especially on deep fading channels. In my experiments with rate

1/2 sequential Fano decoding, I found I could barely handle 8-9%

symbol errors on a hard-decision binary channel. But if I introduced

erasures rather than errors, I could tolerate nearly 50% erasures.

One way to implement erasure decoding even when you don't have a

3-level channel is to send a packet twice and compare the two. You

erase any symbols that don't match.

Phil

------------------------------------------------------------------------

[1-19-95]

[I sent this a few days ago, but it seems to have vanished in

cyberspace... I guess because of the listserver changeover.  Try, try

again... ]

I only tuned into this mailing list quite recently, so I'm just starting

to catch up on some of the discussions.  This message caught my eye:

> Phil's intriguing proposal for using M-ary orthogonal FSK

> with Walsh functions.  Over Christmas break (I'm pursuing

> an MSEE; if only I could catch it) I did some crude

> simulations of Phil's scheme using MATLAB.

>

> First, I generated a 64x64 Hadamard matrix (whose rows are

> Walsh functions).  Then I generated 64 equal amplitude

> sinusoids, each separated by 32 Hz, from 512 Hz to 2528 Hz.

> I then picked a row at random from the Hadamard matrix to

> determine which sinusoids to sum together.  I sampled the

> time sequence at 32 kHz and plotted it over a full period

> of the combined signal.

>

> I used several different Walsh functions to get a feel for

> the general behavior of these signals.  Since I started with

> 64 tones, and only 1/2 are on for any particular Walsh

> function, I was combining 32 separate tones.  The results

> were rather interesting, and very similar to Johan's.

>

> Thus wrote Johan, KC7WW:

>

> > As a way to learn more about how OFDM works for the

> > 39-tone MIL-STD-188 parallel modem, I wrote a C program

> > that simulates 39 parallel tones and combines them into

> > a single output signal.

>

> [snip]

>

> > I also did capture the simulated output waveform and was

> > quite concerned after inspecting the plot  -  the

> > presence of large spikes of significant magnitude (in

> > some cases, some 10-15 dB over the RMS value).

>

> My simulation produced signals with peak-to-RMS ratios of

> 9 dB, they were generally of low amplitude except where

> the sinusoids came into phase and produced a large peak,

> and they generally looked 'noiselike'.  I also noticed that

> the signal's peak amplitude was not symetric about the time

> axis.  The signals had zero average value but the largest

> positive-going peak was twice as high as the largest

> negative-going peak.

Are you sure about the 9 dB?  A 32-tone signal should have a peak-to-RMS

ratio (aka "Crest Factor") of 18 dB for the case in which all of the

tones have zero initial phase shift.  The formula for the Crest Factor for

N tones is:

CF = sqrt(2N), or in dB, 20 log CF.

> As several others have noted, this high peak-to-RMS ratio

> will limit the amount of energy per symbol.  A signal such

> as this is potential trouble in any amplifier unless care is

> taken to control the peak level.  The asymetry in peak

> heights may furthur limit the amount of power an amplifier

> could deliver.  This is definitely not a constant envelope

> signal so linear amplifiers are in order.  As more tones are

> added the signals will have larger peak-to-RMS ratios.  This

> is consistent with Johan's observations since he was summing

> more tones.

>

> The FCC limits amateurs based on PEAK envelope power (PEP),

> not RMS power.  As Phil noted, coding gains can buy back

> most of the loss in energy per symbol caused by the low RMS

> value of the signal, but it's still a significant limitation.

> Disregarding interference, it's ultimately the energy per

> bit that determines the performance of a digital system.

>

> Phil's proposal was for a noncoherent system, which got me

> thinking (scary, eh?).  A noncoherent system ignores the

> phase of a signal; so, we have a free parameter to play with.

> My initial simulation started each sinusoid at zero phase.

> If we could use the phase to reduce the peak value of the

> signal we could get more energy per symbol and get the coding

> gain as well.

>

> First I tried a linear shift in the initial phase across all

> 64 sinusoids.  I started with zero initial phase at the lowest

> frequency and increased the phase linearly to two pi at the

> highest frequency.  This resulted in signals with about the

> same peak-to-RMS ratios (8.25 dB), still 'noiselike', but the

> amplitude peaks became symetrical about the time axis

> (interesting and unexpected).

According to the reference I have (see end of this message), the CF

remains at sqrt(2N) in the case of phase shifts which vary linearly with

frequency, so it should still be 18 dB.

> My next attempt was to assign a random initial phase to

> each of the 64 sinusoids. The initial phase was distributed

> uniformly between zero and two pi.  This produced signals with

> an average peak-to-RMS ratio of 4.75 dB, the signals looked

> extremely 'noiselike', and the amplitude peaks were nearly

> symetrical.  Progress!

I think this is again off by a factor of two, but it illustrates that

random phase is certainly a better choice... in this case, the

dependency of CF on N is of the order of sqrt(log N) instead of sqrt(N).

> These were just the first ways I thought of to change the

> phase, there was no method to my maddness.  Even a naive

> method like ramdomizing the phase produced a 4 dB reduction

> in the peak-to-RMS ratio.  These results lead to a conjecture

> and several questions.

>

> My conjecture is that we can reduce the peak-to-RMS ratio

> further, and maybe even approximate a constant envelope

> signal, by properly choosing the phases of the sinusoids.

> I think this is plausible since a wideband FM signal is a

> constant envelope signal and its spectrum looks like a set

> of nearly equal amplitude sinusoids, within a limited

> bandwidth.  I doubt we could get a truly constant envelope

> signal because we are using a small set of harmonically

> related sinusoids.

Your conjecture is correct. :-)

> Some questions I have:

>

>    -  Are there systematic methods for choosing the

>       initial phases of the sinusoids to minimize the

>       peak-to-RMS ratio?

Yes.  See the paper by Boyd referenced below.  A proof exists that, in

the case where N is a power of 2, a set of phases can be found which

yields a Crest Factor of under 6 dB for arbitrarily large N.  Moreover,

he gives a procedure for selecting phases which yields a CF of about 4.6

dB for *any* N (except when N is very small).  The waveform generated by

this procedure is fascinating: it closely resembles a chirp signal (constant

envelope swept frequency, sometimes used as a spread spectrum technique).

>    -  What limits are there on reducing the peak-to-RMS

>       ratio?

According to Boyd, there is evidence that the ultimate limit is around 3

dB, but no systematic method for finding the phases exists (yet).  A

colleague of mine did some work a few years back on numerical methods

for finding near-optimum phases for multitone modems.  He used an

example of a 12-tone FSK modem (i.e., 12 simulaneous narrow-shift FSK

signals with regular frequency spacing - one of the CCIR standards) and

was able to find phase choices which yielded a CF in the 4-5 dB range

for all 4096 possible frequency combinations.  This would be a

considerable improvement over the 13.8 dB CF you'd get with linear

phases.

>    -  Are these limits, if they exist, related to the

>       number of sinusoids or the Walsh function used?

The limits seem to be independent of N, as long as N exceeds some

minimum number (of the order of 10-20).

>    -  Is this a well-known (or obscure) problem that

>       already has a solution?

It's on the obscure side, but certainly some work has been done.  I

haven't been involved in HF data comms for quite a few years, so I don't

know what recent advances might have been made in multitone modems...

but I can ask around a bit.

>    -  Is reducing the peak-to-RMS ratio really as

>       important as I seem to think it is?

I'd say it's certainly worth further investigation... with the processing

power now available in DSP's, control of the CF seems a lot more

feasible than it used to be.  Sometimes higher average power will buy

you an improvement in BER, and sometimes it won't (when you've hit an

irreducible error rate due to intersymbol interference from multipath),

but overall it would be a win.

> The MIL-STD modems apparently use PSK on the tones as well.

> Since the phase of any particular sinusoid in my simulation

> was constant over the symbol period this should allow using

> methods such as DPSK on the tones if someone wants to add an

> additional layer of modulation.  Would this increase the

> peak-to-RMS ratio?

I'm pretty sure it would, since you're now putting an additional

constraint on the phases.  If you want to Keep It Simple, stick with

noncoherent FSK (which generally works at least as well as DPSK in

multitone HF modems anyway).

References:

S. Boyd, "Multitone signals with low crest factor", IEEE Trans. on

Circuits and Systems, vol. CAS-33, no. 10, October 1986, pp. 1018-1022.

S. Shlien, "Minimization of the Peak Amplitude of a Waveform", _Signal

Processing_, vol. 14, no. 1, January 1988, pp. 91-93.

> Eric, N2NNP

Barry

--

Barry McLarnon  VE3JF/VA3TCP

Ottawa Amateur Radio Club Packet Working Group

Email: bm@hydra.carleton.ca  or bm@lynx.ve3jf.ampr.org

-----------------------------------------------------------------------

Postings are not necessarily in chronological order nor are threads intact or from the same sources.